TEST

PRODUCT

QUALIFICATION

REPORT

TITLE:

LT1912 MSOP Test Platform Migration from

LTX TS88 to Eagle ETS88

PCN NUMBER:

21_0066

REVISION:

A

DATE:

03/01/2021

SUMMARY

The LT1912 is an adjustable frequency (200kHz to 500kHz) monolithic step-down switching regulator that accepts input voltages up to 36V. A high efficiency 0.25Ω switch is included on the die along with a boost Schottky diode and the necessary oscillator, control and logic circuitry. Current mode topology is used for fast transient response and good loop stability. The LT1912 allows the use of ceramic capacitors resulting in low output ripple while keeping total solution size to a minimum. The low current shutdown mode reduces input supply current to less than 1µA while a resistor and capacitor on the RUN/SS pin provide a controlled output voltage ramp (soft-start). The LT1912 is available in 10-pin MSOP and 3mm × 3mm DFN packages with exposed pads for low thermal resistance.

To augment test manufacturing during increase in test volumes, the LT1912 is planned to be

transferred to another test platform, from LTX TS88 to Eagle ETS-88.

There is no change to the form, fit, function, quality, or reliability of the transferred parts.

This report documents the successful completion of the platform migration/ product test transfer requirements for the release of LT1912 from LTX TS88 to ETS-88.

Test product qualification was performed according to Analog Device’s Specification

(TST00094/TST00095 – Test Platform Migration Specification)

TEST AND PRODUCT INFORMATION

Device: 	LT1912

Package: 10-Pin MSOP packages with exposed pads for low thermal resistance.

Part Name:  LT1912EMSE#PBF

Catalog Part Name: LT1912EMSE#TRPBF

Tester Platform1: ETS-88

Handler1: RASCO_1000_RM_AHC

Tester Platform2: LTX TS88

Handler2: RASCO_1000_RM_AHC

TEST DESCRIPTION AND RESULTS (Test Platform Migration Criteria)

Table 1 and Table 2 provide a description of the correlation qualification conducted for the LT1912

platform migration and their corresponding results. For Table 3, small lots were tested on LTX and ETS-88 platforms to validate the LTX TS88 and ETS-88 has the Bin 1 and failure performance. The resulting data were then processed and subjected to the requirement criteria. If all tests passed all criteria, then the platform migration is successfully correlated

Table 1. Mean Shift

| Generic | Package | Tester | LTX (lot size) | ETS-88 (lot size) | Mean Shift [(delta mean)/(limit range) < 5.0%] |
| --- | --- | --- | --- | --- | --- |
| LT1912 | 10-Pin MSOP | LTX and ETS88 | 150 | 146 | Passed |

Table 2. Sigma Spread

| Generic | Package | Tester | LTX (lot size) | ETS-88 (lot size) | Sigma Spread [(sigma new)/(sigma original) < 1.3] |
| --- | --- | --- | --- | --- | --- |
| LT1912 | 10-Pin MSOP | LTX and ETS88 | 150 | 146 | Passed |

Table 3. Yield Validation and Reject Verification

| Generic | Package | Lot | Lot Size | Good units passed on both test platforms? | Validation of rejects passed? |
| --- | --- | --- | --- | --- | --- |
| LT1912 | 10-Pin MSOP | 1 | 150 | Yes | Yes |
| LT1912 | 10-Pin MSOP | 2 | 150 | Yes | Yes |
| LT1912 | 10-Pin MSOP | 3 | 149 | Yes | Yes |

CONCLUSION

The correlation exercise has been successfully completed. The Eagle ETS-88 has been

proven to be at par with the LTX TS88 test platform. It is therefore suitable for release to production

awaiting customer approval.

APPROVALS

Technical Review Board (TRB)

REFERENCE

Technical Review Board: TRB #62958

ADDITIONAL INFORMATION

Homepage: https://www.analog.com/