| Test Procedure | EV-21562-AUTO |
| --- | --- |

Document No.:	?????

Title:	EV-21562-AUTO Customer Evaluation Board Test    Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| A | N/A | Initial Release | 05/24/22 | Chad Wentworth |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
| HW Engineer | David Harrington |

EV-21562-AUTO Testing Procedure

| Part Number: | EV-21562-AUTO | Description: | ADSP-21562 Auto board |
| --- | --- | --- | --- |
| Date: | 05/24/2022 | Written by: | Chad Wentworth |
| Test Fixture: | Laptop | SW – Initial/Date: |  |
| Approved by: |  | HW – Initial/Date: |  |
| Computer: |  |  |  |

- Document Revision History

| Revision Number | Date | Description | Engineer |
| --- | --- | --- | --- |
| A | 05/24/2022 | Initial release | Chad Wentworth |

- Required Equipment

  - (Qty. 1) 3ft USB A MALE TO USB C MALE CABLE

  - (Qty. 2) 3.5mm Female to 3x Male extension cable

  - (Qty. 2) 3.5mm Male to Male adapter

  - 12V 1.5A power supply

- Required software/files

  - EV-21562-AUTO POST.zip file

  - DebugAgentFlashProgrammer utility

  - ADI CCES version 2.11.0 or later

  - Termite Terminal Emulator

- Setup

  - 1 Switch and Jumper Settings

•	EV-21562-AUTO (DUT) Boot mode switch (SW1) is set to ON, OFF

•	EV-21562-AUTO (DUT) P7 OFF

  - 2   Connections

- Connect USB-C cable to the UART (P2) on EV-21562-AUTO and the other end into laptop.

- Attach single end of the 3.5mm Male to 3x Male extension cable to P4, then attach the other 3 ends to P15, P13, and P3.

- Attach single end of the 3.5mm Male to 3x Male extension cable to P5, then attach the other 3 ends to P16, P14, and P12.

- Connect the 12V power supply to P1 of the EV-21562-AUTO

- Programmed Parts

  - Loading EEPROM

- Open the DebugAgentFlashProgrammer utility.

- Click on the “EEPROM” tab.

- Under “Targets”, click the “All” box.

- In the “Configuration” drop-down tab, select EV-21562-AUTO and click “Write”.

- In the “Write to EEPROM?” confirmation box, click “Yes”.

- Verify that the following information is displayed.

  - Programming Serial Flash with POST – U3

- Extract all of the files from the previously installed “manufacturing” zip folder to the desktop.

- Connect USB-C cable to the UART (P2) on EV-21562-AUTO and the other end into laptop.

- To load Serial flash

  - Navigate to the “POST” folder.

  - Double click on the 21562_POST.bat file.

  - Make sure there were no errors during programming.

  - Press any key to exit.

- Open a Termite terminal session and configure the serial terminal with the following settings: COMxxx (Not COM3), 9600 BAUD, Data bits: 8, Parity: none, Stop bits: 1, Flow control: None, Append LF selected, Font: monospaced

- Tests

Standard Loop Tests (3 Loops)

- Press the Reset button (SW2) on the EV-21562-AUTO DUT.

- In the Termite terminal text window, type 1 and press enter.

- Follow the instructions in the Termite window

- Verify that the test passed and in the Termite terminal text window and press a few characters that end with the letter “z” and press “Enter”.

- The Standard Loop tests will continue, without user input for 3 loops.

- Verify that there were no failures when the POST stats are displayed at the end of the 3 loops.

- Remove power from the EV-21562-AUTO.

- Programming AWE BSP(This has to be done last)

- Connect USB-C cable to the UART (P2) on EV-21562-AUTO and the other end into laptop.

- Connect ICE-2000 to P1 and be sure the cable is connected between the ICE-2000 and the PC

- To load Serial flash

  - Navigate to the “AWE” folder.

  - Double click on the 21562_AWE.bat file.

  - Make sure there were no errors during programming.

- Press any key to exit.