Table VII. 8XC51 Code for Reading from the AD7712

MOV SCON,#00010001B;	Configure 8051 for MODE 0			Operation
 MOV IE,#00010000B; 	Disable All Interrupts
 SETB 90H;	Set P1.0, Used as RFS
 SETB 91H;	Set P1.1, Used as TFS
 SETB 93H; 	Set P1.3, Used as A0
 MOV R1,#003H;	Sets Number of Bytes to Be Read in
	A Read Operation
 MOV R0,#030H;	Start Address for Where Bytes Will
	Be Loaded
 MOV R6,#004H; 	Use P1.2 as DRDY
WAIT: 
 NOP; 
 MOV A,P1; 	Read Port 1
 ANL A,R6; 	Mask Out All Bits Except DRDY
 JZ READ;	If Zero Read 
 SJMP WAIT; 	Otherwise Keep Polling
READ: 
 CLR 90H; 	Bring RFS Low
 CLR 98H;	Clear Receive Flag
POLL:
 JB 98H, READ1	Tests Receive Interrupt Flag
 SJMP POLL
READ 1:
 MOV A,SBUF;	Read Buffer
 RLC A; 	Rearrange Data
 MOV B.0,C;	Reverse Order of Bits
  RLC A; MOV B.1,C; RLC A; MOV B.2,C;
  RLC A; MOV B.3,C; RLC A; MOV B.4,C;
  RLC A; MOV B.5,C; RLC A; MOV B.6,C;
  RLC A; MOV B.7,C;
  MOV A,B;
  MOV @R0,A;	Write Data to Memory
  INC R0;	Increment Memory Location
  DEC R1	Decrement Byte Counter
  MOV A,R1
  JZ END	Jump if Zero
  JMP WAIT	Fetch Next Byte
END:
 SETB 90H	Bring RFS High
FIN:
 SJMP FIN