Table V. 8XC51 Code for Reading from the AD7713

MOV SCON,#00010001B;	Configure 8051 for MODE 0

MOV IE,#00010000B;	Disable All Interrupts

SETB 90H;	Set P1.0, Used as RFS

SETB 91H;	Set P1.1, Used as TFS

SETB 93H;	Set P1.3, Used as A0

MOV R1,#003H;	Sets Number of Bytes to Be Read in

A Read Operation

MOV R0,#030H;	Start Address for Where Bytes Will

Be Loaded

MOV R6,#004H;	Use P1.2 as DRDY

WAIT:

NOP;

MOV A,P1;	Read Port 1

ANL A,R6;	Mask Out All Bits Except DRDY

JZ READ;	If Zero Read

SJMP WAIT; 	Otherwise Keep Polling

READ:

CLR 90H;	Bring RFS Low

CLR 98H;	Clear Receive Flag

POLL:

JB 98H, READ1	Tests Receive Interrupt Flag

SJMP POLL

READ 1:

MOV A,SBUF;	Read Buffer

RLC A;	Rearrange Data

MOV B.0,C;	Reverse Order of Bits

RLC A; MOV B.1,C; RLC A; MOV B.2,C;

RLC A; MOV B.3,C; RLC A; MOV B.4,C;

RLC A; MOV B.5,C; RLC A; MOV B.6,C;

RLC A; MOV B.7,C;

MOV A,B;

MOV @R0,A;	Write Data to Memory

INC R0;	Increment Memory Location

DEC R1	Decrement Byte Counter

MOV A,R1

JZ END	Jump if Zero

JMP WAIT	Fetch Next Byte

END:

SETB 90H	Bring RFS High

FIN:

SJMP FIN