Table VI. 8XC51 Code for Writing to the AD7713

MOV SCON,#00000000B;	Configure 8051 for MODE 0

Operation & Enable Serial Reception

MOV IE,#10010000B;	Enable Transmit Interrupt

MOV IP,#00010000B;	Prioritize the Transmit Interrupt

SETB 91H;	Bring TFS High

SETB 90H;	Bring RFS High

MOV R1,#003H;	Sets Number of Bytes to Be Written

in a Write Operation

MOV R0,#030H;	Start Address in RAM for Bytes

MOV A,#00H;	Clear Accumulator

MOV SBUF,A;	Initialize the Serial Port

WAIT:

JMP WAIT;	Wait for Interrupt

INT ROUTINE:

NOP;	Interrupt Subroutine

MOV A,R1;	Load R1 to Accumulator

JZ FIN;	If Zero Jump to FIN

DEC R1;	Decrement R1 Byte Counter

MOV A,@R;	Move Byte into the Accumulator

INC R0;	Increment Address

RLC A;	Rearrange Data—From LSB First

to MSB First

MOV B.0,C; RLC A; MOV B.1,C; RLC A;

MOV B.2,C; RLC A; MOV B.3,C; RLC A;

MOV B.4,C; RLC A; MOV B.5,C; RLC A;

MOV B.6,C; RLC A: MOV B.7,C; MOV A,B;

CLR 93H;	Bring A0 Low

CLR 91H;	Bring TFS Low

MOV SBUF,A;	Write to Serial Port

RETI;	Return from Subroutine

FIN:

SETB 91H;	Set TFS High

SETB 93H;	 Set A0 High

RETI;	Return from Interrupt Subroutine