Appendix

IAP Demo on ADuC7026 Evaluation Board

- Open IAP_boot Project, and make sure ADuC7026 memory have been Mass Erased.

Open PC->Start->All Programms-> Accessories-> Communications-> HyperTerminal, and change the Baud rate to 115200.

- Click “Start/Stop Debug Session”, and then insert a breakpoint on line 147 of main.c file where is about “senddata(0xB);”, and then press F5.

- In HyperTerminal, click Transfer -> Send Text File…

- Choose IRQ0_App\BIN\Send Enable Upgrade CMD.txt

- Program will stop at line 147 of main.c, and you press F5

- Then HyperTerminal will receive a “B” character.

- Choose IRQ0_App\BIN\ Send First 256 byte Counter.txt

- Choose IRQ0_App\BIN\ First 256 Bytes from HEX.txt

- Then HyperTerminal will receive a “B” character.

- Choose IRQ0_App\BIN\ Send residual 200 byte Counter.txt

- Choose IRQ0_App\BIN\ residual 200 Bytes from HEX.txt

- Then HyperTerminal will receive a “A” character.

- Then you press Halt to stop program. You could use D 0x81000, 0x811FF in command or Memory Window to check the memory content of address from 0x81000 to 0x811FF, to see if user program has been downloaded via ADuC702x IAP program.

Be note that this IAP program is just to provide a method how to realize IAP function, and customer should re-write it and modify communication rule according to your own requirement.