# AD9379, AD9378-1, and AD9378-2 SERIALZER IBIS AMI MODEL PACKAGE DESCRIPTION

## OVERVIEW

The  analog_devices_AD9379_AD9378-1_AD9378-2_serializer_ami_r1_p0.zip file contains several files used to model the AD9379_AD9378-1_AD9378-2 deserializer using IBIS AMI. The included files are appropriate for simulation with Quantum Channel Designer from SiSoft, ADS from Keysight, as well as other channel simulators that support the IBIS specification 5.1, and they have been tested with QCD and ADS.

To model the AD9379, AD9378-1, and AD9378-2 accurately, the package model (package/AD9379_AD9378-1_AD9378-2_deserializer_package_model.s16p) must be instantiated between the IBIS AMI Tx and the channel.

## FILES CONTAINED IN PACKAGE

The purposes of the files contained in analog_devices_AD9379_AD9378-1_AD9378-2_deserializer_ami_r1p0_release_customer.zip are described in Table 1.

Table 1. File descriptions

| File Name | Description |
| --- | --- |
| doc/ | Certification report and Correlation report for the original model delivered from SiSoft. |
| ibis/ | This directory contains everything a user will need to run these models in any IBIS-AMI compliant simulator. It contains the IBIS file, .ami file, Linux and Windows executables, and on-die termination touchstone files. The .ibs file should be selected in the simulator, and it will reference the other files in the directory. |
| package/AD9379_AD9378-1_AD9378-2_serializer_package_model.s16p | This touchstone file is used for the package. It must be placed between the IBIS AMI model instantiation and the channel in the simulation. The port descriptions are contained in the comments in the beginning of the file. |

## Correlation

The AD9379, AD9378-1, and AD9378-2 have de-emphasis settings of 0-3. These are mapped to the AMI parameter deemph settings of 0,2,3,5 as shown in Table 2. The curve overlay metric used as a figure of merit is over 93% for each Corner and de-emphasis combination tested, which is better than the target value of greater than 90%. Also, comparing the waveforms visual in Figure 1 to Figure 12 for ADS and Figure 13 to Figure 24 for QCD shows a reasonable correlation between the model and lab data.

Table 2. IBIS AMI parameter map and curve overlay metric

| Corner | Deemph | IBIS AMI Deemph Setting | QCD Curve Overlay Metric | ADS Curve Overlay Metric |
| --- | --- | --- | --- | --- |
| Typical | 0 | 0 | 94.8% | 95.5% |
| Typical | 1 | 2 | 94.7% | 94.2% |
| Typical | 2 | 4 | 94.3% | 93.7% |
| Typical | 3 | 5 | 94.6% | 93.7% |
| Fast | 0 | 0 | 96.2% | 95.8% |
| Fast | 1 | 2 | 96.1% | 95.5% |
| Fast | 2 | 4 | 95.7% | 95.1% |
| Fast | 3 | 5 | 95.9% | 95.1% |
| Slow | 0 | 0 | 96.3% | 95.4% |
| Slow | 1 | 2 | 96.2% | 94.9% |
| Slow | 2 | 4 | 95.7% | 93.9% |
| Slow | 3 | 5 | 95.9% | 94.0% |

Figure 1. Corner=Typical Deemph=0

Figure 2. Corner=Typical Deemph=1

Figure 3. Corner=Typical Deemph=2

Figure 4. Corner=Typical Deemph=3

Figure 5. Corner=Fast Deemph=0

Figure 6. Corner=Fast Deemph=1

Figure 7. Corner=Fast Deemph=2

Figure 8. Corner=Fast Deemph=3

Figure 9. Corner=Slow Deemph=0

Figure 10. Corner=Slow Deemph=1

Figure 11. Corner=Slow Deemph=2

Figure 12. Corner=Slow Deemph=3

Figure 13. Corner=Typical Deemph=0

Figure 14. Corner=Typical Deemph=1

Figure 15. Corner=Typical Deemph=2

Figure 16. Corner=Typical Deemph=3

Figure 17. Corner=Fast Deemph=0

Figure 18. Corner=Fast Deemph=1

Figure 19. Corner=Fast Deemph=2

Figure 20. Corner=Fast Deemph=3

Figure 21. Corner=Slow Deemph=0

Figure 22. Corner=Slow Deemph=1

Figure 23. Corner=Slow Deemph=2

Figure 24. Corner=Slow Deemph=3