# Introduction

This command line executable tool generates a Verilog module which implements the JESD204 receive transport layer [Ref. 1]. The user specifies in a configuration file one or more modes to be supported by the transport layer module. These modes are defined as a set of JESD204 parameter values:  L, M, F, S, N', and CF. The transport layer converts JESD204 lane data output from a JESD204 link layer IP to a data bus with a fixed width, containing interleaved virtual converter samples. Both JESD204B and JESD204C link layers are supported.

The following figure illustrates a top-level system into which the generated transport layer module is instantiated.

# Module Port Map

The generated Verilog module has the following ports:

| Name | Direction | Size | Description |
| --- | --- | --- | --- |
| clk | input | 1 | Clock |
| reset | input | 1 | Reset, active high, synchronous |
| jesd204_l | input | (1) | JESD204 L parameter - 1 (2) |
| jesd204_m | input | (1) | JESD204 M parameter - 1 (2) |
| jesd204_f | input | (1) | JESD204 F parameter - 1 (2) |
| jesd204_np | input | (1) | JESD204 N' parameter - 1 (2) |
| lane_data | input | (3) | Input JESD204 lane data lane_data is a concatenation of JESD204 lane data. Each lane of data is input_octets_per_lane*8 bits wide, and the number of lanes is equal to the largest number of lanes requested across all modes in the configuration. The least-significant octet of each lane’s input corresponds to the earliest received octet on the physical interface. If using Xilinx JESD204B link layer IP, the output of the link layer IP does not need to be endian swapped. If using Xilinx JESD204C link layer IP, the output of the link layer IP DOES need to be endian swapped. The least-significant octet of each lane’s data is swapped to the most-significant octet etc. If the current mode has less than the maximum number of lanes, then the most-significant bits corresponding to the unused lanes are discarded. |
| in_valid | input | 1 | Input data valid indicator |
| out_data | output | (4) | Output JESD204 sample data out_data is a concatenation of interleaved virtual converter samples. Regardless of the current value of the JESD204 parameters inputs, the entire out_data bus contains valid data during each cycle in which out_valid is asserted. If the JESD204 M parameter, the number of virtual converters, is > 1, then the virtual converter samples are interleaved by one sample per virtual converter. If output_sample_octets is non-zero, then each sample is padded to the multiple of output_sample_octets*8 bits greater than or equal to the JESD204 N' parameter value of the current mode. If output_sample_octets is zero, then no padding is performed. |
| out_valid | output | 1 | Output data valid indicator |

Notes:

- JESD204 parameter input ports are sized to be able to represent the largest value of the parameter across all specified modes in the configuration. If the configuration specifies only one mode, these ports are not present.

- JESD204 parameter input port values are specified as the parameter value minus one

- lane_data port width is the max number of lanes * input_octets_per_lane * 8 bits

- out_data port width is output_data_octets*8 bits

# Configuration File Format

An example configuration file is provided with the tool. The format of the configuration file is illustrated in Figure 1. The first section of the configuration file specifies system-level parameters. These parameters are dependent upon the input and output formats for the blocks connected to the transport layer module in a system. The second section specifies a set of JESD204 modes which the transport layer must support.

Figure 1: JESD204 Transport Layer Generation Tool configuration file format

# Output Format Example

The format of the output data on the out_data signal is defined in the Module Port Map section.  Examples of one cycle of output data for different configurations are given below. VC stands for virtual converter.

In this example, M=2, N’=16, and output_data_octets=8, and output_sample_octets=0 or 2 (no padding needed for this mode).

In this example, M=1, N’=12, output_data_octets=8, and output_sample_octets=2.

# Procedure

Extract the zip file

Modify adi_jesd204_tl_rx.cfg to match the requirements of your system

Open a Windows cmd shell window.

Change directory to the location at which the files were extracted

Run the command: adi_jesd204_tl_rx.exe adi_jesd204_tl_rx.cfg > <out_file_name>.sv

Replace <out_file_name>.sv in the preceding command with a different file name to give the generated file a different file name.

<out_file_name>.sv is created based on adi_jesd204_tl_rx.cfg.

Instantiate the generated module and connect to the output of the JESD204 link layer IP.

# References

- JEDEC STANDARD Serial Interface for Data Converters JESD204B

# Revision History

| Date | Version | Changes |
| --- | --- | --- |
| 6/2/2020 | 1 | Initial version |