# Overview

The aim of the project is to develop a system that can be controlled from a host that integrates standard Ethernet communication capabilities, such as a personal computer with an RJ45 interface, and is able to interact with multiple remote devices, such as stepper motors or sensors.

The idea is that remote devices can be placed at considerable distances from the host, and (up to a pre-defined power consumption value, decided during design phase) can also be phantom powered using only a power supply located near the host. To meet these requirements, the chosen communication protocol is 10BASE-T1L Ethernet, which allows for communication using a single twisted pair over distances that can reach up to 1km. It is also capable of power delivery on the same twisted pair using the Power over Data Line (PoDL) standard.

On the software side, the interface for the host is written in Python language for compatibility with multiple platforms, and the code that runs on the microcontroller that manages the interaction with remote devices is written in C using the ChibiStudio toolchain for STM32 platform. To keep embedded code simple and lightweight, the implemented communication protocol is based on plain Ethernet frames with a predefined format and does not rely on additional stacks for upper layers.

The first remote device integrated in the system is a stepper motor controlled by the Trinamic TMC5160 stepper motor controller and driver. The choice of this particular IC is due to the fact that it already integrates a motion controller and can be programmed via UART or SPI interface to operate in both position and velocity control modes, requiring minimum interaction with the microcontroller at runtime. Moreover, it is also able to manage acceleration e deceleration ramps without requiring explicit step time calculations from the microcontroller. This device is able to communicate in both the directions with the host, because it can operate as an actuator, taking commands from the computer and controlling the motor accordingly, and as a sensor, by replying to the host with speed, position and load angle of the motor (taking advantage of the StallGuard2 feature).

Figure 1 High-level scheme of the system

# Hardware

## EVAL-ADIN1100EBZ

This board is used as a media converter between the interface from the host device, which uses a standard 10BASE-T Ethernet link with RJ45 connectors, and the 10BASE-T1L link. With its default firmware can be used in different modes of operation, the one used for this project is mode 15, that allows it to work as a transparent bridge between the two links: frames are received by the on-board microcontroller using the ADIN1200 PHY and are forwarded on the T1L link through the ADIN1100 PHY, without any changes in the data. The board can also be connected via USB used as a virtual COM port to visualize debug data and other information about the status of the link.

By changing configuration of jumper J1 (BOARD PWR SELECTION) it is possible to select as main power supply for the board between the USB connector and an external power supply that can be applied to the terminal blocks or to the DC barrel plug. The latter is particularly useful to power the receiving device using Power over Data Line.

## EVAL-ADIN11X0DBZ

Figure 2 EVAL-ADIN11X0DBZ scheme (DRAFT)

EVAL-ADIN1100 and EVAL-ADIN1110 boards include three headers (unpopulated by default) that can be used to add PoDL capability to the system. The board used to implement this functionality is the EVAL-ADIN11X0DBZ, that can be directly plugged on these headers.

This board can be configured in two different ways.

Only R1 and R2 soldered: the board is configured to superimpose the voltage provided by the external power supply to the 10BASE-T1L data line. A voltage must be present at the terminal blocks of the main evaluation board, and its power selection jumper must be set to EXT.

Only R1A and R2A soldered: the board is configured to take power form the data line and to supply the board with it, the rectifier is added for reverse polarity protection.

If the application requires powering external devices from the EVAL-ADIN1110 board, a small modification to the standard configuration is needed for the receiving EVAL-ADIN11X0DBZ board: resistor R1 is also soldered, to bring the voltage also to the power terminal block of the main board, that in this way can be used as an output. Current delivered is limited to 0.5A by the fuses present on the evaluation boards, and the voltage cannot exceed 24V due to the TVS diodes used as protections for input power lines.

To evaluate total losses and voltage drops along the power supply lines, voltage was measured at critical nodes, while the board was powered with a 24V supply and the motors were enabled and standstill (current consumption measured on power supply wiring ranging from 190 to 220 mA). Results are reported below:

Figure 3 Overview of PoDL circuit and losses (DRAFT)

The major contribution to voltage losses over the line is due to the bridge rectifier present on the EVAL-ADIN11X0DBZ, and the Schottky diode D4 placed in series of Arduino Vin line. Other significant drops are also due to coupled inductors used on the PoDL link.

## EVAL-ADIN1110EBZ

This board is mainly composed by the ADIN1110 10BASE-T1L transceiver and an STM32L4S5QII6 microcontroller, that is connected through an SPI interface and some GPIOs to the transceiver. The board is used to receive data sent over the T1L link, and thanks to the Arduino compatible headers to manage the communication with controlled devices, such as the motion controllers. The board also provides an USB connector, that is used to allow the communication with one of the UART peripherals of the microcontroller. Using the provided pushbuttons, it is possible to boot the microcontroller in bootloader mode, and to flash a custom compiled firmware through the onboard USB connector and the STM32CubeProgrammer software in UART mode.

Board power supply is chosen through the BOARD PWR SELECTION header, and for this application the setting that allows the board to be powered from data line is EXT+. Moreover, the motion controller board used in this project can be powered from the “Vin” pin of the Arduino-compatible headers to avoid additional wiring. To allow this operation the jumper ARD must be closed. In the default configuration of the board the Vin pin is used as an input. To allow the operation as an output, the board has been modified by soldering diode D4 in the opposite direction. NOTE: when the board is powered through USB connector for programming, ensure that ARD jumper is off, otherwise it will try to power the motion controller board from the 5V supply provided by the USB port.

A DIP switch (S201) could be used to modify default settings for the ADIN1110 transceiver, but in this application, they are completely managed and by the microcontroller.

## TMC5160-EVAL (1st version)

The board used in the first implementation of the project to interface the microcontroller with the TMC5160 stepper motor controller is the TMC5160-EVAL, which provides access to all the pins used to configure and control the IC. The board also integrate input capacitors and the output power stage required to drive the motor, as the IC allows for the use of external MOSFETs. The device is operated as a “full featured motion controller and driver”, meaning that the microcontroller does not have to generate STEP and DIRECTION signals, but can be programmed using a serial interface. The chosen interface for this application is the single wire UART, because during the first tests the development board used to interface with the devices only exposed a single SPI interface, that was already reserved for the communication with the ADIN1110. The use of multiple nSS lines (active low Slave Select) was avoided since it would require software management of the GIPOs, adding more dead times to the ADIN1110 communication, while a single line can be fully managed in hardware by the SPI peripheral.

| Pin number | Signal name | Description |
| --- | --- | --- |
| 1 | +VM | Positive supply for the motor, 8…55V. |
| 2 | GND |  |
| 3 | GND |  |
| 5 | +5V_USB | Supply for VCC_IO pin of the TMC5160. The onboard regulator (IC202) is bypassed by soldering R212 and leaving R211 open. It can be directly connected to a GPIO of the microcontroller (standard 3.3V level) and can be used as a hardware reset pin (active low) for the IC. Requires to be actively driven high with ~3mA current capability to properly operate the driver. |
| 8 | DRV_ENN | Enable for driver power stage. Not used, tied to GND. |
| 9 | ENCN_CFG6 | Not used, tied to GND. |
| 20 | SD_MODE | Tied to GND, STEP/DIR interface is disabled |
| 22 | SPI_MODE | Tied to GND, UART mode is enabled |
| 23 | CLK16 | Tied to GND, internal clock source enabled |
| 32 | SP1_SDI_CFG1 | Tied to GND. In UART mode, is used as Next Address Input (NAI) to set device address. Low level means address 0, high level address 1. Used if more than 1 unit is connected on the same UART line. |
| 37 | SWP_DIAG1 | Single wire UART data output. Requires an external pull-up (in this application a 2.7k resistor to VCC_IO line is used). (Note: the internal, software selectable pullup resistor of the microcontroller is too weak for the application) |
| 43 | GND |  |
| 44 | GND |  |

Table 2 Used pins of the TMC5160-EVAL, connector X101

To simplify the connection with the EVAL-ADIN1110 and interface board made on a breadboard has been prepared, that provides terminal blocks for motor power supply and expose control pins on a header.

## TMC5160 SilentStepStick shield (2nd version)

A second version of the interface with motion controllers, both in hardware and software, has been developed to add the possibility of controlling multiple motors, following the idea of a possible integration of the system with a more complex mechanical system, such as a multiple axis cartesian robot or a simple robotic arm. This introduced also the necessity of being able to synchronize precisely multiple motion commands, that was not easily allowed using the 1-wire UART bus. Moreover, the idea was also to simplify the whole setup, reducing the number of wirings and different modules needed.

The communication interface was switched from 1-wire UART to SPI to allow for the synchronization of different controllers. By using common data bus and separate chip select lines, motion parameters can be programmed individually on each driver, and when the programming phase is finished all the motion can be started simultaneously by setting and clearing all the chip select lines at the same time.

The new board is based on the Arduino Shield form factor, that can be mounted as a plug-in module for the EVAL-ADIN1110 board eliminating the necessity of wiring between the two boards. To keep the board compact, while leaving the possibility to easily replace the drivers and allowing the manufacturing of the prototype on a breadboard, the TMC5160 SilentStepStick evaluation boards are also used, and a single shield can interface with two of them. The prototype of this board is built on the Screw Shield R3, a prototyping shield by Adafruit designed for Arduino UNO boards that already includes male-female headers to allow stacking of multiple boards. It also exposes Arduino pinout on standard 3.5mm screw terminal blocks, to ease wiring with external devices. In this application, some modifications were made to allow the use of these screw blocks to connect stepper motors to the board. The used pinout and form factor also permits stacking of multiple interface boards, increasing the number of motors possible to control. However, if PoDL supply is used also for the motors, the number of motors is limited to 2 due to the maximum current capability for the link of 0.5A. With an independent power supply on the remote device side of the system this number can be increased, up to the limit given by the number of free GPIOs of the microcontroller that can be used as chip select line.

Figure 4 Overview of the complete system

Figure 5 Schematic drawing of the prototype shield (DRAFT)

The schematic above shows all the connections between the TMC5160 SilentStepStick boards and the Arduino compatible headers. One important note is that the screw blocks provided with the proto shield are used for the connections with the phases of the motors. The first motor is connected to the W, X, Y, Z terminals, that have no corresponding pin on the Arduino headers, while the second motor connects to terminals A0, A1, A2 and A3, that by default should be connected to the Arduino header reserved for pins with A/D conversion capability. To ensure that no contact could happen between these pins the phases of the motor, the corresponding header on the prototyping shield was left unmounted on purpose.

Another important feature of the board is that it includes electrolytic capacitors close to all power supply pins of the SilentStepStick boards, that are used to smoothen current peaks on the power supply lines and to reduce the power supply current requirements during the inrush transient that occurs when motors get energized. The following screenshots, taken from the oscilloscope using a current probe, show the comparison between the current profile with and without capacitors on the supply lines.

Figure 6 Current profile on input power supply lines, single TMC5160 SilentStepStick, no capacitors, 24V supply. Horizontal 10ms/div, Vertical 500mA/div. Peak consumption reaches 2A. (DRAFT: lo screen è fatto con il telefono, e non sono sicuro sia quello corretto)

Figure 7 Current profile on input power supply lines, single TMC5160 SilentStepStick, with 470uF capacitor, 24V supply. Horizontal 10ms/div, Vertical 500mA/div. Peak consumption is below 500mA. (DRAFT: lo screen è fatto con il telefono, e non sono sicuro sia quello corretto)

# Software

## Overview

Communication between the host and remote devices is based on raw ethernet frames, without a standard protocol to manage upper layers of the stack to keep the code simple and lightweight.

| Layer | OSI Model | Protocol |
| --- | --- | --- |
| 3…7 | Network…Application | Nonstandard protocol, managed by adopting a fixed structure for frames payload that is parsed in the same way by the host and the remote device |
| 2 | Data Link | Ethernet Data Link |
| 1 | Physical | Ethernet 10BASE-T1L |

Table 3 Used stack for communication between devices (DRAFT)

On the host side, frames are generated by a Python script based on the Scapy module, that provides a set of commands for monitoring and sending packets over network interfaces. Packets can be created and manipulated at each layer of the stack, so it also includes tools to build raw Ethernet frames by specifying the content of their fields, such as MAC addresses and the payload.

For the remote device, a different approach is needed: the ADIN1110 transceiver offers an array of 32-bit register that can be used as a transmit buffer, and it is up to the microcontroller firmware to build the packet accordingly to the Ethernet standard.

To ease debug process, the traffic can be monitored in real time using the Wireshark protocol analyzer on the host device.

Figure 8 Standard Ethernet frame format

Destination and Source MAC fields are dependent on communication direction. The MAC address of the EVAL-ADIN1110 has been fixed to 00:e0:22:fe:da:c9 (as used in the default evaluation firmware), while the source address can very depending on the interface used by the host. Since no ARP protocol is implemented, the remote device is not able to automatically identify the MAC address of the host, so all the frames are transmitted from the microcontroller with broadcast address. When the EVAL-ADIN1110 is receiving data, frame filtering is only based on the destination address, so host MAC address plays no role in the communication.

The Ethertype field can be used with two different functions: if the value is below 1500, it represents the length of the payload data. If it is above 1536 (hex: 0x0600), it is used to identify which protocol is encapsulated in the payload.

Checksum is automatically appended at the end of the frame by both Scapy and the ADIN1110.

Payload field is used to exchange data between devices. The minimum length required for this field to complete a valid Ethernet transmission is 46 bytes.

## Data exchange

Each data transaction is started by the host. The payload field of the Ethernet frame is organized in segments with a fixed length (46 bytes), so only payloads with lengths that are multiple of 46 bytes are considered valid. If the data to be transmitted does not utilize all the segment, the remaining bytes are padded with zeros. Each segment has a 2-byte, fixed format header and a 44 bytes payload.

Figure 9 Payload and segments structure

The Device type field is used to determine how the content of the data field will be processed, and what actions the microcontroller has to perform when the frame is received (e.g., changing the velocity of a motor or answering to the received frame with some data). Each device type is represented with an unsigned integer, ranging from 0 to 255.

The other field, Device ID, is an 8-bit value that is used to allow the identification of multiple occurrences of the same device type, for example when multiple motion controllers are connected to the same microcontroller. IDs cannot be arbitrarily assigned, in both the host and the remote microcontroller software are automatically assigned starting from zero with increments of 1. Currently supported device types are listed in the following table.

| Code | Name | Description |
| --- | --- | --- |
| 0 | Generic Device | Not implemented, used only for testing purposes |
| 1 | Motion Controller | Includes information necessary to operate a physical motion controller (control mode, speed and position setpoints), such as the TMC5160. |
| 2 | Generic Remote | Information request segment, empty data field. The remote device answers with a similar segment that also includes requested data. |

Table 4 Implemented devices list

### Motion Controller

Figure 10 Motion Controller frame format

| Field | Length (bytes) | Description |
| --- | --- | --- |
| Status | 1 | 0 = Motor disabled 1 = Motor enabled |
| Command | 1 | 0 = No command 1 = Ramp (position) mode 2 = Constant speed mode |
| Direction | 1 | 0 = Clockwise 1 = Counterclockwise Note: only used in ramp control mode |
| Speed | 4 | Unsigned 32-bit integer value. |
| Target position | 4 | Signed 32-bit integer value. |

Table 5 Motion controller frame fields description

### Generic Remote

Figure 11 Generic remote device query segment

Figure 12 Generic remote device answer segment

This segment is used to request data from the remote device. It is not linked to any physical device, and the payload can be used to exchange arbitrary information. Each ID can be configured to handle answer data differently.

## Host interface

Host interface is written in Python language and utilizes the Scapy module to manage the communication with the Ethernet interface. It is composed of two different files, Devices.py and Main.py.

Devices collects the classes that represent the “Device Types” included in the communication protocol. They all follow the same structure:

- Declaration of the constants used by the class. The first one identifies the device type and corresponds to the ID assigned in the “Device Type” field of the corresponding segment. Other constants can be related to the meaning of fields, such as the convention for directions in the motion controller.

Figure 13 Declaration of the constants for a class

- Class constructor, in which all the properties of the class, that will correspond to the fields of the relative segment, are created and initialized with their default values. Also, the address of the device is assigned as a parameter passed to the constructor.

Figure 14 Motion controller class constructor

- Methods for modifying the properties of the object. The idea is to create a high-level set of functions to control the remote device, instead of having to directly modify value of the variables.

Figure 15 Methods for changing motion direction

- “packSegment” method. This method is present at the end of each class, and returns a list composed of 46 bytes that corresponds to the full segment, including also the header, that can be sent to the remote device. If data is not enough to fill the full segment, remaining bytes are padded with zeros.

Figure 16 packSegment method

Main instead is the file that can be executed to test the system. It is composed of two parts, the first is the declaration of the “Controlled devices” class. This class includes some parameters used for the communication, such as the MAC address of the remote device and the Ethernet interface on which the communication can be established. MAC address is fixed to the value found in the default firmware of the EVAL-ADIN1110, but the interface must be chosen correctly depending on the host device.

Note: a possible upgrade would be automatic detection of the correct interface. Idea: before the connection has been established, the remote device periodically sends an empty frame with the correct MAC address. The host scans all the available interfaces, and for each one listens for packets with the correct MAC address and payload. When the corresponding frame is found, it selects the interface, sends an ACK to the remote device and the communication can be established.

Manual procedure for finding correct interface on a Windows host:

- Ensure the RJ45 cable of the device is not connected to the computer.

- Run command prompt (Win+R, type cmd and then press OK).

- Run ipconfig, the list of the available interface should appear.

- Connect the cable to the RJ45 port of the computer and re-run ipconfig. The new list should show an additional interface, that corresponds to the device just connected to the computer.

The constructor of the class is used to initialize a list for each device type supported by the application. Initially, all the lists are empty. A set of functions is provided to add an object corresponding to a device to each collection. The function also initializes the address of the device using consecutive numbers, to ensure consistency with the structure used in the firmware.

Figure 17 “ControlledDevices” class constructor and initialization functions

The class also includes methods to send the segments corresponding to a device over the link. Communication on the Ethernet interface is based on the Scapy module, that allows to build, send and receive packets at different levels of the TCP/IP stack. In this application packets are sent and received at level two, corresponding to Ethernet Data-Link. The following functions are utilized:

myPacket = Ether(dst=self.ADIN1110MacAddress, type=len(framePayload))/Raw(framePayload)

Ether initializes the Ethernet header of the frame, specifying the destination MAC address “dst” and the Ethertype field value “type”, that is used to specify the length in bytes of the frame. The payload is appended to the frame by using the operator “/” and is initialized as Raw class. The constructor of this class accepts list of types corresponding to bytes.

sendp(myPacket, self.ADIN1110NetIface, verbose = False)

sendp is the function used to send a frame at Ethernet Data-link level. Parameters are the packet to send, created previously, and the chosen interface. Parameter verbose is used to suppress function output in the terminal.

myAnswer = sniff(count=1, iface = self.ADIN1110NetIface, timeout = 1, filter = "ether src host " + self.ADIN1110MacAddress)

sniff function is used to listen the link and report received frames by returning a list of frames. Count parameter is used to define the number of frames that have to be received before returning the result of the function. Iface is the interface on which the device is communicating. Being a blocking function, also the timeout parameter is specified, in seconds. If no frames compliant with the filters are found, this function returns a zero-length list when the timeout is expired. Filter is as string that specifies how the frames should be filtered, following the Berkeley Packet Filer (BPF) format. In this case, “ether src host <device address>” allows the application to receive only frames that are sent with a source MAC address corresponding to <device address>.

Figure 18 Example of methods used to interact with remote devices. The first one sends a one-way command, while the second also waits for an answer.

The last part of this file includes a main program, that can be used to demonstrate functionality of the system. It initializes the classes as required, sends a command to the motion controller accordingly to the specified parameters and subsequently polls the remote at a pre-defined rate to get updated values for motor parameter, such as speed, position, and load angle. (Note: the answer is structured in this way because on both host and remote devices the Generic Remote Device with address 0 is used to exchange data about the motion controller).

## Firmware

### Overview and microcontroller configuration

The firmware for the microcontroller that receive messages over the 10BASE-T1L link and elaborates them was originally written for the SDP-K1 development board, based on an STM32F469NIH6 microcontroller, and then ported on the EVAL-ADIN1110 on-board microcontroller (STM32L4S5QII3P). The code is written in C language using the Chibi Studio environment. A custom board configuration file, based on the template for STM32L4xx microcontrollers has been prepared, that contains the initialization of all used pins (as reported in the table in paragraph “Pin assignment”). This file is found in the “…/chibios<version>/os/hal/boards/” directory, under the ADI_EVAL_ADIN1110 folder, along with the .c/.h pair that can be generated starting from it, that includes all the defines and initialization functions used to configure the microcontroller. This folder is common to all projects developed for this board.

Other microcontroller configuration files can be found under the “.../cfg/” folder in the project directory and include the settings that could change more frequently between projects. In particular, the “mcuconf.h” file includes the settings for the clock tree of the microcontroller, that is particularly useful when calculating register values for the configuration of timers and peripherals.

The project is based on a copy of the demo project that can be found under “…/chibios<version>/demos/STM32/NIL-STM32L476-DISCOVERY/”. This project already includes the initialization of ChibiOS/NIL, a lightweight version of the fully featured ChibiOS/RT real time operating system.

NOTE: I was not able to find the complete procedure for creating a new project for a custom board. I followed the procedure explained in this video (https://www.youtube.com/watch?v=HwJqZfITvV0), but when compiling the code multiple errors appeared, related to some .h files that did not correspond to the chosen microcontroller. These are the additional steps I had to take to get the program to compile (may not be complete):

- Makefile, ‘HAL-OSAL files’ section. Row 103 is modified to point to the STM32L4xx+ folder, as from ST Microelectronics website this microcontroller is under the ‘L4+’ series.

- In the project directory, folder ‘cfg’. Added ‘#define STM32L4S5_MCUCONF’ along with the microcontrollers already present.

- (This point requires modifying a file that could be shared between different projects, I think there may be a smarter way to do it). From the project explorer tree, open ‘os>hal>ports>STM32L4xx+>stm32_registry.h’. Assing FALSE value to:

  - STM32_HAS_SDMMC1

  - STM32_HAS_GPIOI

  - STM32_HAS_GPIOJ

  - STM32_HAS_GPIOK

### Pin assignment

| Pin name | Peripheral | Signal name | Header | Notes |
| --- | --- | --- | --- | --- |
| PA0 | UART 4 | TMC5160_DATA | ARDUINO D1 | Used for bidirectional communication in one wire mode, must be configured as Alternate Function with Open-Drain output. External pull-up required, not used in SPI configuration. |
| PA1 | GPIO | TMC5160_nRST | ARDUINO D0 | TMC5160 VCC_IO supply |
| PA9 | UART 1 | USB_UART_TX | - | VCP over USB connector |
| PA10 | UART 1 | USB_UART_RX | - | VCP over USB connector |
| PB5 | GPIO | CSn_2 | ARDUINO D9 | Chip select line, controller 2 |
| PB6 | GPIO | CSn_3 | ARDUINO D8 | Chip select line, controller 3 (not used) |
| PB11 | GPIO | T1L_INTn | EXT HOST INT_N | ADIN1110 interrupt pin |
| PB12 | SPI 2 | T1L_CSn | EXT HOST CSN | ADIN1110 SPI interface |
| PB13 | SPI 2 | T1L_SCLK | EXT HOST SCLK | ADIN1110 SPI interface |
| PB14 | SPI 2 | T1L_MISO | EXT HOST MISO | ADIN1110 SPI interface |
| PB15 | SPI 2 | T1L_MOSI | EXT HOST MOSI | ADIN1110 SPI interface |
| PC7 | GPIO | T1L_RESETn | EXT HOST RST_N | ADIN1110 reset pin |
| PC10 | SPI 3 | ARDUINO_D13_SCK | ARDUINO D13 | TMC5160 SPI interface |
| PC11 | SPI 3 | ARDUINO_D12_MISO | ARDUINO D12 | TMC5160 SPI interface |
| PC12 | SPI 3 | ARDUINO_D11_MOSI | ARDUINO D11 | TMC5160 SPI interface |
| PC13 | GPIO | LED_uC0 | - | - |
| PE2 | GPIO | LED_uC1 | - | - |
| PE6 | GPIO | LED_uC2 | - | - |
| PG12 | SPI 3 | CSn_1 | ARDUINO D10 | Chip select line, controller 1 |
| PG15 | GPIO | LED_uC3 | - | - |

Table 5 EVAL-ADIN1110 microcontroller pin assignment

### Clock configuration

Clock configuration is done in the “cfg>mcuconf.h” file. It follows the same structure that is reported in the microcontroller reference manual, and that is more conveniently represented in STM32CubeMX software.

Figure 19 First section of the clock tree, from input reference signal to HCLK

/*

* HAL driver system settings.

*/

#define STM32_NO_INIT                       FALSE

#define STM32_VOS                           STM32_VOS_RANGE1

#define STM32_PVD_ENABLE                    FALSE

#define STM32_PLS                           STM32_PLS_LEV0

#define STM32_HSI16_ENABLED                 FALSE

#define STM32_LSI_ENABLED                   TRUE

#define STM32_HSE_ENABLED                   FALSE

#define STM32_LSE_ENABLED                   TRUE

#define STM32_MSIPLL_ENABLED                TRUE

#define STM32_MSIRANGE                      STM32_MSIRANGE_4M

#define STM32_MSISRANGE                     STM32_MSISRANGE_4M

#define STM32_SW                            STM32_SW_PLL

#define STM32_PLLSRC                        STM32_PLLSRC_MSI

#define STM32_PLLM_VALUE                    1

#define STM32_PLLN_VALUE                    80

#define STM32_PLLP_VALUE                    7

#define STM32_PLLQ_VALUE                    6

#define STM32_PLLR_VALUE                    4

#define STM32_HPRE                          STM32_HPRE_DIV1

#define STM32_PPRE1                         STM32_PPRE1_DIV1

#define STM32_PPRE2                         STM32_PPRE2_DIV1

#define STM32_STOPWUCK                      STM32_STOPWUCK_MSI

…

Figure 20 Extract from mcuconf.h file, represents the clock settings in the firmware. The ones used for the project are highlighted

As can be noted above, the MultiSpeed Internal (MSI) is used as clock source and is configured to 4MHz. The corresponding input of PLL Source Mux (PLLSRC) is selected, and PLLM1, PLLN and PLLR values are set to provide an output frequency of 80MHz. Using the STM32_SW define, the System Clock Mux is configured to take as input the frequency generated by the previous PLL. In this way SYSCLK frequency is set to 80MHz, and with STM32_HPRE define the AHB prescaler is set to /1, giving a HCLK frequency of 80MHz. This frequency is supplied to APB1 and APB2 buses, used by al the peripherals and clocks in the microcontroller. It can be further reduced by setting PPRE1 and PPRE2 to values different than /1.

### Main

The main.c file includes the configuration of all the peripherals of the microcontroller and the initialization of the drivers for the devices connected to the board. The board is programmed with the bootloader over UART mode, so real-time debug is not possible, unless an external debug probe such as an ST-LINK is connected to the JTAG header. The same UART peripheral (UART1) is used to print debug messages, that are delimited by “#if ENABLE_DEBUG_SERIAL … #endif” lines, so that can be easily disabled to speed up code execution.

Figure 21 Debug message print function example

Main function is only used to start peripheral drivers and to start the real-time os, and all device-related initialization and update functions are managed by a thread. A second thread is used to blink a green LED, to have a quick way to determine if a software fault has occurred.

The first initialized device is the ADIN1110 transceiver.

Figure 22 ADIN1110 configuration

For the ADIN1110, the ADIN1110_Config function initializes the object that corresponds to the device, taking as input a pointer to the object that will represent the device and a struct that includes configuration parameters, such as related GPIO lines and SPI driver. Then, a hardware reset is performed, to ensure that every time the microcontroller resets also connected devices are reconfigured in the correct way. The ADIN1110_Setup_Link function is used to establish communication over the T1L link with another device, that in this case could be the EVAL_ADIN1100. The function is structured as a state machine and returns only when the link is established (otherwise, if the connection cannot be completed, the function will keep trying to establish the link. A possible upgrade could be adding a configurable timeout or maximum number of tries, that when expires returns and an error and stops code execution). If the link is established correctly, also LINK/ACTIVITY LEDs (LED_0 and LED_1) should light up on the board.

Figure 23 TMC5160 configuration

Following the ADIN1110 configuration, TMC5160 motion controllers are also initialized. The TMC5160_Config function initializes the corresponding objects as was done for the ADIN1110, and performs a hardware reset of the device by cycling the line connected to the VCC_IO pin. If a common VCC_IO line is used for all the devices, as in this project, all TMC5160_Config functions must be called before doing other operations on the motion controllers, otherwise the last TMC5160_Config call will bring back all the devices to reset state.

The “TMC5160_Set_Phase_Current_RMS” function is used to apply a global scale factor to the phase current set at hardware level using the sense resistor (RSENSE), according to the values specified in the configuration struct for the device, and if needed must be called prior to any other operation, to avoid motor overheating. This value is stored in the device until next reset. Then, COOLCONF and CHOPCONF registers are configured accordingly to the datasheet, and before staring any operation the TMC5160_Set_Dynamic_Current function is used to specify what fractions of the full current (that already includes the global scale factor applied before) must be used during standstill and motion operations. Both the values can go from 0 to 31 and correspond to fractions from 1/32 to 32/32 of the full current.

Figure 24 Infinite loop in the main thread

The infinite while loop in the same thread contains the code that is continuously executed when the program is running. Each iteration starts with detecting if new frames are available in the RX FIFO queue, by checking the frame_available flag in the ADIN1110 struct, that is updated on interrupt as soon as a frame is received. Then, function ADIN1110_Read_Frame is used to copy into a variable the content of the RX FIFO, and Parse_Command_Frame extracts data from the received frame payload, according to the format adopted by the communication protocol. Eventually, function Update_Devices is used to start the commands specified in the received frames.

### Drivers

Drivers for external devices can be found in the project directory, under the “Usr” folder, as a pair of source and include files.

#### ADIN1110

This driver includes functions used to communicate with the device over the SPI interface and the definition of required custom types.

The ADIN1110_CFG_typedef is a structure used to organize data needed to configure the ADIN1110 object. Reset and Interrupt pins are specified as lines (type used by ChibiOS to store GPIO port name and pin number within a single variable). The other required parameter is the driver of the SPI.

The ADIN1110_typedef includes all the data needed by driver functions to communicate with the device, and also stores received frames and values of control and configuration registers. It must be passed to the functions as a pointer.

Figure 25 Low-level communication functions

The above functions implement the read and write procedures described in the datasheet. The chosen mode is standard SPI, with no CRC at the end of each transaction. Two types of read or write operation are possible:

The one used by Read32 and Write32 functions, allows writing or reading single 32-bit registers. They are particularly useful for device configuration and debug or monitoring operations. Each operation is started by sending ‘0b10’ bits, followed by the R/W bit, register address and an optional turnaround field, followed by register data or dummy clock cycles while listening the MISO line in case of a read operation. If large amounts of data have to be transmitted, this mode add considerable overhead.

Burst mode, instead, utilizes the same header, but data can be transmitted or received in any multiple of 4 bytes. This mode is used when reading or writing data over the T1L link with minimal overhead.

NOTE: The datasheet does not provide information about the Turn Around (TA) field. From an online article: “The Turn Around field is an 8-SCLK cycle separation between the data you send in the SDI, and when the ADIN1110 starts sending data on the SDO. You can set this TA field to be all 0.” (Full post: https://ez.analog.com/industrial-ethernet/physical-layer-devices/f/q-a/552045/adin1110-spi-clarification-turn-around-field-spi-packet-clarification)

Figure 26 MDIO access functions

MDIO (Management Data Input/Output) is a two-wire serial bus specified in the Media Independent Interface (MII) standards, used to read and write control and configuration register on Ethernet PHY devices, along with lines dedicated to data transmission. The ADIN1110 does not expose an MDIO interface to manage the integrated PHY, but achieves this functionality through a fixed subset of standard registers (MDIOACCn) that can be read or written using SPI. Following the procedure found in the datasheet, this register set can be used to access to both PHY registers that follow clause 22 or clause 45 standards. The above functions implement these procedures for clause 45 registers, and only require the PHY device address and register address to complete a transaction. Both values are found in the “PHY CLAUSE 45 REGISTER MAP” in the datasheet.

All other functions in the driver are based on this set of lower-level communication functions and provide a more convenient way to perform different actions. For example, this additional set of function is used to send or receive raw Ethernet frame over the T1L, without requiring any explicit interaction with registers.

Figure 27 Frames management functions

Another important function needed to correctly receive frames on the device is the ADIN1110_Add_Filter function. By default, no filters are set, and all the frames received by the ADIN1110 are discarded. A maximum of 16 filters can be programmed to decide which frames should be forwarded to the host, and which ones must be discarded. Filters are based on the Destination MAC field of the received Ethernet frame and are defined by two different 6-byte values that are passed to the function as arrays of six uint8_t elements. The “address” argument specifies the address to compare with the Destination MAC field in the received frame, while the “mask” argument must be interpreted as a binary value. The bits set to ‘1’ are compared to determine if the frame has to be accepted, while bits set to ‘0’ are ignored. In this way, classes of multiple MAC address can be filtered using a single filter. Note: this function only allows adding filters that can are used to forward corresponding frames to the host, but they can also be configured to discard matching frames or to split received frames between a high-priority and low priority RX queues.

Figure 28 Add filter function

Configuration of the device is accomplished by calling the ADIN1110_Setup_Link, that implements the procedure described in the “BRINGING UP 10BASE-T1L LINKS” paragraph of the ADIN1110 datasheet.

#### TMC5160

NOTE: older versions of the project include drivers to communicate with TMC5160 devices using UART interface. Then, to allow for precise synchronization between different motion controllers, the choice of the interface was switched to the SPI.

The TMC5160_SPI driver includes functions used to perform configuration of the TMC5160 motion controller, and to operate the device in the “full featured motion controller” mode, allowing for constant speed control with acceleration and deceleration ramps and position control following six-point ramps. The device shares the same register map for both the interfaces, but while with the UART integrity of the transaction is checked with a CRC byte for each message, using SPI the mechanism is based on the retransmission of written data (only for write accesses).

As for the ADIN1110, A TMC5160_CFG_typedef struct is defined, which contains all parameters needed to configure the device. Two floating point values, fullCurrentRMS and targetCurrentRMS, are specified for deciding maximum operating phase current of the motor. fullCurrentRMS must correspond to the RMS current value set by the Rsense resistors, while targetCurrentRMS is the maximum current that the motor can handle (these parameters set a global scale factor to the maximum allowable motor current, but standstill and motion current can be further tuned). The idea behind this driver is that multiple motion controllers can be operated over the same SPI bus, and individually addressed by using separate Chip Select (CSn) lines. SPI peripheral handler, Chip Select line and Enable line are also specified in the configuration struct.

Figure 29 TMC5160 configuration struct typedef

When the TMC5160_Config function is called, a TMC5160_typedef object is initialized with parameters specified in the config struct. The pointer to this object is then used to refer to the physical TMC5160 device when calling other functions.

Another useful struct for the operation of the TMC5160 driver is the TMC5160_RAMP_PARAMS_typedef. This struct includes all the parameters needed to define the profile of the six-point ramps used when the driver is configured in positioning mode. An Excel spreadsheet has been created to verify if ramp settings are plausible, to determine the positioning time for the programmed motion and to ease conversions between register units and physical values.

Figure 30 Speed profile of the six-point ramp

Figure 31 Ramp configuration structure

Vstart defines the height of the initial velocity step (y coordinate of point 1).

A1 is the acceleration used in the first phase (slope of 2-1 segment).

V1 is the velocity used to split the two constant acceleration phases (y of points 2 and 5).

Amax is maximum allowable acceleration, used in the second acceleration phase (slope of 3-2 segment).

Vmax is the velocity of the constant speed phase (y of 4-3 segment).

Dmax is maximum allowable deceleration (slope of 5-4 segment).

D1 is the deceleration used during the last phase (slope of 6-5 segment).

Vstop defines the height of final velocity step (y of point 6).

tZeroWait is the time the driver has to wait before starting a new ramp, for example when motion direction is changed.

TMC5160_COOLCONF_typedef struct is used for the configuration of the COOLCONF register, that allows enabling CoolStep and StallGuard2 features of the driver. NOTE: the idea is that when a complex register has to be configured, similar structs and corresponding functions can be created to improve code readability and to ease parameters tuning.

Figure 32 Low level communication functions

Two low level communication functions are used to perform basic reading and writing operations on device registers, addressing a single TMC5160. The device is selected by passing the correct pointer to a TMC5160_typedef object along with register address and data to be written or a pointer for storing the result of the read operation.

To determine if a write operation was successful, the TMC5160 mirrors back written data to the host during the following train of 40 clock pulses. The TMC5160_SPI_Write32_register function also listens for this answer and compares it with sent data. If the operation was successful, a ‘1’ value is returned. The read function, on the other hand, does not include any form of error detection.

An important note is that the SPI peripheral used for the communication with multiple TMC5160 devices cannot be configured with hardware Chip Select line management, as this mode can handle a single line. Chip Select pins must be configured as standard GPIOs in output mode, and their level is manually cleared before starting each transaction and set when it is finished.

Figure 33 Other TMC5160 functions

Other functions, based on lower-level ones, are provided to allow control of the motion without having to deal with registers. Four modes of operation, through the TMC5160_SetRampMode_* functions can be chosen:

VelPositive and VelNegative modes are used to start a constant velocity motion in both the directions. Velocity can be modified with Vmax register. When transitioning between two different velocities, the value set in the Amax register is used for both acceleration and deceleration phases. Relevant parameters for this type of motion can be modified using the TMC5160_Set_Vmax and TMC5160_Set_Amax functions.

Hold mode is used to keep motor velocity constant until another ramp mode is selected.

Position mode is used for precise positioning, following six-point ramp profiles. The TMC5160_Ramp_Config function is used modify ramp parameters accordingly to the struct passed as a parameter and SetTargetPosition and SetActualPosition can be used to program motion extents.

To allow for synchronized motion between different motion controllers on the same SPI bus, an additional function is defined to enable low-level communication with multiple devices at the same time. This is accomplished by selecting a list of motion controllers, and by setting and clearing simultaneously their Chip Select lines during write operations.

Figure 34 Broadcast register write on two different devices.

Ch1 (yellow): SPI Clock; Ch2 (blue): CSn_1; Ch3 (pink): CSn_2.

Figure 35 Synchronization list entry typedef

To obtain synchronized motion, an array of TMC5160_SYNC_MOTION_ENTRY_typedef must be declared. Each element includes a pointer to one of the motion controllers that need to be synchronized, and its corresponding ramp parameters and target position.

Figure 36 Synchronization functions

The Broadcast_Write32_register function is used to perform write operations on multiple devices simultaneously, while the Start_Sync_Motion is used to start a synchronized motion following the instructions found in the TMC5160_sync_list array that is passed as a parameter.

### Remote devices interface

The last pair of .c/.h files included in the “Usr” directory provides an interface between the commands received on the T1L link and the physical devices connected to the board. The high-level idea behind this module is that a set of structs, one for each device type, is defined with fields that correspond to the ones used in the communication protocol. Every time a frame is received, it is split into segments and processed. If a device is correctly recognized, the corresponding object and its fields are updated. Then, the pointer to this device is added to a list of devices with pending updates, and when all the segments in the received frames have been processed another function is called, that scans the list and performs the actions required by each entry.

In the Devices.h file are defined the structs corresponding to devices, and the number of each device used in the application, that must correspond to the one defined in the Python interface.

Figure 37 Number of declared devices for each type

Figure 38 Example of a device struct (motion controller)

The additional “updateAvailable” field is used as a flag, set to ‘1’ when a new command is received and there is a pending action for the device.

Also, the typedef for the elements of the list of devices with pending updates is defined in this file. The device pointer is a void pointer to allow the use of a single list for all device types, but an additional field to specify device type is needed to correctly handle the pointer in update functions.

Figure 39 Update list entry typedef

The following functions are defined to manage the processing of the received commands:

Figure 40 Command processing functions

The Parse_Command_Frame takes as input the frame received by the ADIN1110. After determining the number of segments included in the payload, for each segment device type and device address are identified. Then, a switch-case loop based on the device type unpacks segment data into the fields of the corresponding device struct and sets its updateAvailable flag to 1. This loop also adds the device pointer to the list of devices that have pending updates and updates the variables that stores the length of the list.

The Update_Devices function is called after the Parse_Command_Frame has returned. It iterates through all the elements of the updates list, and a switch-case loop is used to determine which function to execute when updating a particular device type. Once finished, it resets the length of the list to zero.

For each device type, a function is defined to perform required actions, based on the fields of the corresponding device objects that has been already updated. This function also defines the correlation between each device object and the physical device that it controls. For example, in the case of motion controllers, an initial switch loop is used to determine which motor controller is selected for the operation.

Figure 41 Correlation between device objects and physical motion controller peripherals

Each of these functions, before returning must set the updateAvailable flag to zero, indicating that the required action has already been executed.

An exception to this standard process is done for synchronized motion control: when a position command is detected, instead of executing it immediately, the Update_Motion_Controller function adds an entry to a synchronization list, specifying target position and motion parameters. Only when all the update requests have been processed (ensuring that all position commands have been added to the synchronization list) the Update_Devices function checks if the length of the synchronization list is not zero and starts a synchronized motion accordingly.