====================================================================

A D V A N C E D   C I R C U I T S

ORDER FORM

VOICE  800-289-1724      303-576-6983  MODEM (>=9600 BAUD)

FAX  303-576-6993      303-289-1248  MODEM (<=2400 BAUD)

INTERNET ADDRESS .... 102203.3505@compuserve.com

====================================================================

YOUR COMPANY NAME: Linear Technology Corporation

SHIP TO ADDRESS: 1630 McCarthy BLVD.

CITY, STATE, ZIP: Milpitas, CA 95035

PHONE NO: (408) 954-8400

FAX NO: (408) 944-9410 or (408) 428-9413

BUYER'S NAME: Denni Vicente.....(408) 428-2073

TECHNICAL CONTACT (IF DIFF): Kim Tran..........(408) 954-8400 x 3411

PURCHASE ORDER NO: X20011L

Please place number and the name of KIM TRAN on outside of shipping container, thank you.

BOARD PART NO: DC232A

QUANTITY OF BOARDS: 16 BOARDS ON 4”X16” PANEL

HOW MANY LAYERS: 2           (FAB PRINT IS REQUIRED AND MUST SHOW

ABSOLUTE OVERALL DIMENSIONS: 4” X 16”    (HOLE LOCATIONS BY SIZE AND SHAPE)

# OF SIDES TO SOLDERMASK: 2           (IF ONE, PLEASE SPECIFY WHICH SIDE)

(Y/N) ANY SURFACE MOUNTS?: Y

NUMBER OF SIDES TO LEGEND: 1           (SILKSCREEN)

(Y/N) ANY GOLD?: N           (FINGERS, CONTACTS, ETC.)

NUMBER OF CNC ROUTE POINTS: 0           (PLEASE INCLUDE CUTOUTS, NOTCHES,

(NUMBER OF CHANGES IN X,Y)  0           (CHAMFERS, ETC.)

SHIP HOW: FED-EX PRIORITY OVERNIGHT

FED EX ACCT. NO: 10325746-8

PAYMENT METHOD: Net-30

CREDIT CARD NO: n/a

CREDIT CARD EXPIRATION DATE: n/a                 THANK YOU FOR

NAME ON THE CARD: n/a                 YOU ORDER !

CARD TYPE: n/a

Our "Engineering Prototypes" are intended to test and verify designs.  These

prototypes are NOT guaranteed to meet production specifications.

>>>  PROTOTYPE SPECIFICATIONS  <<<

FR4 .062" MATERIAL, 1 OZ COPPER WEIGHT, TIN-LEAD REFLOW FINISH, 8/8 MIL

TRACE/SPACE MINIMUM, .015 MIL HOLE SIZE MINIMUM - ALL PLATED-THROUGH,

10 CNC ROUTE POINTS, UL APPROVED WET MASK, SILKSCREEN/LEGEND WHITE

NON-CONDUCTIVE INK, NO ELECTRICAL TEST.

TOLERANCES: DRILL +/- .005 : FAB +/- .010.

> PRODUCTION ORDERS WILL ASSUME ALL ABOVE SPECIFICATIONS UNLESS SPECIFICALLY

STATED OTHERWISE.

PROTOTYPE "MANUFACTURABILITY RULES":           RESULT IF VIOLATED:

IF ANGULAR RING < .008"                      YOUR JOB GOES ON HOLD

IF TRACE WIDTH, AIR GAPS < .008"             YOUR JOB GOES ON HOLD

IF SOLDER MASK CLEARANCE < .014" DIAMETER    WE ENLARGE TO .014"

IF LEGEND LINE WIDTH < .008"                 WE ENLARGE TO .008"

IF INNER-LAYER CLEARANCES < .040" DIAMETER   WE ENLARGE TO .040"

THE FOLLOWING IS A CHECKLIST FOR MODEM FILE TRANSFERS:

> ALL FILES ARE "ZIPPED" UP INTO A FILE CALLED "YOUR-PART NO.ZIP"

- ZIP.EXE CAN BE DOWNLOADED FROM MODEM

> THE ZIPPED FILE REQUIRES THE FOLLOWING FILES:

- GERBER FILES (FORMAT 274-D, ONE FILE PER LAYER)

- FABRICATION DATA (GERBER OR CAN BE FAXED)

- APERTURE FILE (FOR EACH LAYER IF NECESSARY)

- CNC DRILL FILE (FORMAT EXCELLON WITH NO ZERO SUPRESSED)

- "READ.ME" FILE (ASCII TEXT INFORMATION FILE)

- THIS ORDER FORM COMPLETELY FILLED OUT, SO ORDER BEGINS IMMEDIATELY

> THE READ.ME NEEDS TO CONTAIN THE FOLLOWING TEXT INFORMATION:

- A LIST OF EVERY FILE NAME, AND DESCRIBE WHAT IT IS

- ANYTHING ELSE YOU NEED TO TELL US TO COMPLETE YOUR ORDER

> ALL THESE ARTICLES WILL INSURE QUICK AND ACCURATE RESPONSE.

EXTRA HINTS FOR CERTAIN CAD PACKAGES TO AVOID CONFUSION AND/OR PROBS.

SOME DESIGN PACKAGES PRODUCE INFORMATION THAT CAN BE MISCONSTRUED.

TO HELP AVOID MANUFACTURING MISHAPS, CONSIDER THE FOLLOWING:

We will view the files from the "COMPONENT"-side looking down, through

the board.  "SOLDER"-side is ALWAYS on bottom.

Strategic placement of targets helps to ensure correct orientation if

each layer.  (i.e. 3 targets, UPPER-LEFT; UPPER-RIGHT; & LOWER-LEFT.)

Including your part number and/or company name on copper layers (Right-

Reading from the respective side of the board) usually prevents

your board form being built inverted. (Top on Bottom, Bottom on Top.)

Board outline (FAB LINES) included with gerbers helps to ensure correct

size and shape.

An accurate FAB PRINT is required to ensure dimensions are followed

correctly.  This will also allow verification of drill sizes.

Do NOT hesitate to include any information that you think may be

Unusual.  THIS MAY PREVENT A DELAY OF SHIPMENT.

"WHAT ELSE MIGHT CAUSE MY BOARD TO GO ON HOLD?"

-Corrupt PKZIP file. (Zip file fails CRC check when unzipped,

files are damaged.)

-Fabrication print not supplied and/or is missing the following:

1. Drill sizes and locations by shape.

2. Overall dimensions (Showing any cutouts, slots, anything unusual, etc.).

3. Material type and finished copper weight and thickness.

4. Tolerance specifications. (Defaults: +/-.005 Drill, +/- .010 Fab.)

(Fab print can be GERBER, or FAX of PLOT.)

-Deviations between Gerber and Fab print.

-Drill file missing or in wrong format.  (Excellon format, no zero supression.)

-Drill has obviously missing holes or incorrect locations.

-Aperture list corrupt or incomplete (Undefined D-codes).

-Aperture list missing.  Text list of D-codes used by each gerber file

is required (Shapes/sizes).

-Gerbers in 274-X format. (We require 274-D format with aperture list

for each layer.)

-Gerber files are scaled.  (We require 1 to 1 size files.)

-Conflicting or missing Right-reading information. (Which side is Up/Down?)

-Thermal connections missing on inner layers (Multi-layer).

-Copper layers with no pads and/or no traces.

-Manufacturability rule violations:

1. Pad/Trace, Pad/Pad, Trace/Trace less than 8 mils.

2. Trace size less than 8 mils.

3. Angular ring is less than 8 mils.

4. Silkscreen legend line width less than 8 mils.

5. Inner layer clearances less than 20 mils per side.