## REQUEST FOR THE SECOND PROTOTYPE FABRICATION

- BOARD SIZE: 2.00” X 2.00”.   0.01” MIN COPPER THICKNESS, 2 LAYERS BOARD. NON ROUTED.

LIST OF GBR FILES:

- ART01.PHO & .REP			LAYER 1 - TOP LAYER

- ART02.PHO & .REP			LAYER 2 - BOTTOM LAYER

- DD0124.PHO & .REP		DRILL DRAWING LAYER (FAB DWG)

- SM0121.PHO & .REP		TOP SOLDER MASK

- SM0228.PHO & .REP		BOTTOM SOLDER MASK

- SMD0123.PHO & .REP		TOP SOLDER PASTE

- SST0126.PHO & .REP		TOP SILKSCREEN

- DRL01.DRL				DRILL FILES

- DRL01.LST				DRILL FILES

- DRL01.REP				DRILL FILES

- PANALIZE 16 BOARDS ON 4” X 16” PANEL ,  0.00” AIRGAP BETWEEN BOARDS.

- MATERIAL: FR4, .062” THICKNESS, 1 OZ CU, TIN-LEAD REFLOW FINISH.

- PLEASE LOOK AT THE FABRICATION DRAWING FOR THE OTHER REQUIREMENTS FOR THE JOB.

- PLEASE CONTACT KIM TRAN AT  408-954-8400 X 3411 OR E-MAIL TO ME AT  Kimtran@aol.com  IF YOU HAVE ANY QUESTION.

THANK YOU

KIM