# LT1675

# 250MHz RGB Multiplexer with Current Feedback Amplifiers

# Description

The LT1675 is a high speed RGB multiplexer designed for pixel switching and fast workstations graphics.  Demonstration Circuit DC240 highlights some of the key characteristics of the LT1675 Multiplexer.  Foremost in importance is the small, 0.5 sq. inch, footprint.  This is accomplished by including 6 gain set resistors internally and using the 16-lead narrow SSOP package.  Included on chip are three SPDT switches and three current feedback amplifiers.  The current feedback amplifiers drive double-terminated 50 or 75 cables and are configured for a fixed gain of 2.  The demonstration board for DC240 has BNC connectors for six inputs and three outputs, as well the Select Input which switches between inputs R1G1B1 or R2G2B2.  Also included is an Enable BNC that is used to shut down the LT1675 to near 0mW.

### Performance Summary

## Parameter				Conditions					   Typical Value

Channel-to-Channel Select Time

Delay Time			Measured from time SELECT Pin crosses Logic Threshold		5ns

Switching Time			Time for Vout to go from 0V to 1V				2.5ns

Small-Signal –3dB Bandwidth		Less than 1dB Peaking					250MHz

Gain Flatness				Less than 0.1dB						70MHz

Slew Rate				Vout=5Vp-p						1100V/s

Channel Select Output Transient		Measured Between Back Termination and Load			50mVp-p

Output Current				Vin=1V, Vo=0V						70mA

#### Operation

6dB of Gain

DC240 exhibits the excellent performance of the LT1675.  This MUX has the internal gain set of +2V/V or 6dB, because it is designed to drive a double-terminated 50 of 75 cable that has an inherent 6dB loss.  To aid in evaluation, three resistors R10, R11, R12 can be added to DC240 to simulate the 75 cable load.  Without these resistors, the outputs of DC240 drives 75 cables directly.

#### Power Supplies

The LT1675 will function with supply voltages below 2V (4V total), however, to ensure a full 1Vp-p video signal (2Vp-p at the output pins), the power supply voltage should be between 4V to 6V.  The LT1675 is designed to operate on 5V, and at no time should the supplies exceed 6V.

#### Expanding Inputs

In video routing applications where the ultimate speed is not mandatory, as it is in pixel switching, it is possible to expand the number of MUX inputs by shorting the LT1675 outputs together and switching with the ENABLE pins.  The internal gain set resistors have a nominal value of 750 and cause a 1500 shunt across the 75 cable termination.  The effect of this loading is to cause a gain error.  For ten LT1675s (20 Red, 20 Green ad 20 Blue) the gain error is only –1.7dB per channel.