Demo Board DC274 Quick Start Guide

Description

Demo board DC274 is a 200kHz 1.5A buck DC/DC switching converter using the LT1576. With its integral power switch, the LT1576 makes a very compact and low parts count DC/DC converter. It runs at 200kHz, allowing the use of all-surface-mount components to make a very small converter. LT1576 uses the current mode control topology for its fast transient response and good loop stability. The saturating power-switch design helps achieve high efficiency. The LT1576 is supplied in a fused-lead SO-8 package. Full cycle-by-cycle short-circuit protection and thermal shutdown are provided. Pin 8 functions as either a shutdown pin (DC274A-A) or a synchronization pin (DC274A-B). A shutdown signal reduces supply current to 20µA. Synchronization allows an external logic-level signal to increase the internal oscillator frequency from 250kHz to 400kHz.

Demonstration circuit DC274 is a complete DC/DC step-down regulator using the LT1576. The input voltage ranges from 5.5V to 25V. The output voltage can be either 5V(jumper J1 open) or 3.3V(jumper J1 closed) at 1.25A. The shutdown/synchronization pin has been brought out for optional use. The LT1576 will find applications in personal computers, disk drives, portable handheld devices, battery-powered systems, battery chargers and distributed power systems. As demonstrated by DC274, an all-surface-mount component solution makes LT1576 ideal for space-conscious DC/DC converter systems.

Test and Measurement Setup

Refer to Figure 1 for proper measurement equipment setup and follow the procedure outlined below:

- Connect the input power supply to the VIN and GND terminals. Input voltage is limited to between 5.5V and 25V.

- Connect an ammeter in series with the input supply to measure input current.

- Connect either power resistors or an electronic load to the VOUT and GND terminals.

- Connect an ammeter in series with the output load to measure output current.

- For the shutdown demo board (DC274A-A), the SHDN terminal should be left floating for normal operation.

- For the synchronization demo board (DC274A-B), the SHDN terminal should be connected to a proper synchronizing signal (refer to the LT1576 data sheet).

- Set the output voltage with jumper J1. With J1 open, the output voltage is 5V. With J1 closed, the output voltage is 3.3V.

- After all connections are made, turn on input power and verify that the output voltage is correct.

Some hidden text

Figure 1. DC274 test and measurement setup