Quick Start Guide:

1. Install the DC285 software on a P.C. and attach the DC157 demo board

to the P.C.'s parallel port (note: the DC285 installs both the DC285

software and the DC157 software).

2. Apply 5V to the DC285 and connect the board to the DC157 via their

5 pin connectors.

3. The operator can now either run the RunDC285 batch file located in

C:\DC285 or enter discrete SMBus commands described below.

4. Enter SMBus commands at the DOS prompt under the DC157 directory

to vary the fan speed or to read fault conditions. The SMBus Write

Command format for the LTC1695 whose "Write" address is E8 is as follows:

Smb w, E8, 'Command Byte'

Examples:

C:\DC157\smb w, E8, 00, 00, 00	sets Vout of the LTC1695 to 0V.

C:\DC157\smb w, E8, 32, 00, 00	sets Vout of the LTC1695 to 2.5V.

C:\DC157\smb w, E8, 3F, 00, 00	sets Vout of the LTC1695 to 4.922V.

Note:	The above examples output an extra word because this is the format

of the DC157 SMBus software commands. The LTC1695 which accepts

Command Bytes simply ignores the Least Significant Word sent by the

DC157 software.

The SMBus Read Command for the LTC1695 whose "Read" address is E9 is as follows:

C:\DC157\smb r, E9

Note:	The earliest version of the DC157 software does not support SMB Byte reads.