# DC369 Introduction

## Description:

Demonstration Circuit DC369 demonstrates the LTC1646’s ability to safely hot swap a CPCI bus requiring 5V and/or 3.3V supplies. The LTC1646 is able to power a wide range of capacitive loads in current limit and uses dual level circuit breakers to protect against overcurrent and short-circuit fault conditions. In addition, the LTC1646 biases bus I/O connector pins to 1V, monitors the state of the 5VOUT and 3.3VOUT supply voltages with the HEALTHY# signal and combines PCI_RST# with HEALTHY# on-chip to generate the LOCAL_PCI_RST# signal.

## Quick Start Guide

Refer to Figure 1 for proper measurement equipment setup and follow the procedure outlined below:

- Connect the 5V input power supply to the +5V and GND terminals on the System Backplane board.

Connect the 3.3V input power supply to the +3.3V and GND terminals on the System Backplane board.

#### Figure 1. DC369A Test and Measurement Setup

- Verify that the BD_SEL# jumper (JP1) is set to the GND position.

- Enable the 5V and 3.3V supplies and insert the DC369 circuit card into the System Backplane connector.

- Verify that the green HEALTHY# LED (D4), red 5V at 5A LED (D1), and red 3.3V at 7.6A LED (D2) are illuminated.

- Connect a voltmeter across the I/O (E3) and GND (E2) terminals to measure the precharge output voltage.

- Connect a voltmeter across the RESET# (E1) and GND (E2) terminals to measure the LOCAL_PCI_RESET# output voltage.

- Connect a voltmeter across the 5V at 5A (J2) and GND (J3) connectors to measure the 5V output voltage.

- Connect a voltmeter across the 3.3V at 7.6A (J4) and GND (J3) connectors to measure the 3.3V output voltage.