# Jumper Assignment

| ### Function | Jumper Position |  |
| --- | --- | --- |
|  | ON | OFF |
| VID0 | VID0=0 | VID0=1 |
| VID1 | VID1=0 | VID1=1 |
| VID2 | VID2=0 | VID2=1 |
| VID3 | VID3=0 | VID3=1 |
| VID4 | VID4=0 | VID4=1 |
| VRON | VRON signal high | VRON signal low |
| DSPLP# | DSPLP# signal high | DSPLP# signal low |
| DPRSLPVR | DPRSLPVR signal high | DPRSLPVR signal low |
| GMUXSEL | GMUXSEL signal high | GMUXSEL signal low |
| DLC ON | Dynamic Load Test Circuit shut off | Dynamic Load Test Circuit turned on |

| ### Function | Jumper Position |  |  |
| --- | --- | --- | --- |
|  | Leftmost | Rightmost | Left Off |
| VBIAS | Onboard 3.3V bias is used | External 3.3V bias is used | For start-up without external 3.3V bias |
| FCB | FCB=0 | FCB=2V | FCB=5V |

# Start-Up Procedure

- Apply 3.3V logic voltage across “+3.3V” and “SGND” terminals, if an external 3.3V bias supply is available. Put “VBIAS” jumper at the rightmost position (and skip Step 5 later). Then jump to Step 3.

- When an external 3.3V bias is not available, leave the “VBIAS” jumper off.

- Select VID code through “VID0” ~ “VID4” jumpers.

- Apply input power across “VIN+” and “VIN-”. Make sure input voltage is 7~24V.

- Put “VBIAS” jumper at the leftmost position.

- Select GMUXSEL/DPRSLPVR/DPSLP# combination wanted.

- Put on “VRON” jumper.

- Apply load across “VCC+” and “VCC-”.

- An external 5V supply can also be connected at “EXTVCC” for higher efficiency.

## To Do Transient Test

- Shut down the running converter by taking “VRON” jumper off.

- Apply 12V supply through “+12V”, “-12V” and “PGND”.

- Put “VRON” jumper on.

- Release “DLC ON” jumper.

- Tune R39/R43/R41 to get wanted transient slope and amplitude.

For more technical help on this, Contact:

David Chen

Applications Engineer

(408) 954-8400 ext 3796

ychen@linear.com