Test Procedure for DC377B

Test should be set up as shown in drawing.

Initial status of jumpers:

- VID4=ON

- VID3=ON

- VID2=OFF

- VID1=OFF

- VID0=OFF

- GMUXSEL=ON

- DPRSLPVR=OFF

- DPSLP#=ON

- VRON=OFF

- DLC ON=ON

- FCB=”L”

- VBIAS=”E”

Test Procedure:

- Turn on VIN bench supply (24V  beware: never beyond 24V!).

- Turn on +3.3V bench supply (3.3V).

- Turn on EXTVCC bench supply (+5V).

- Put VRON jumper on.

- Turn on the bench load across VCC. Set its current to be 0.

- Test #1: reading of DVM1 should be (1.4000.010) V.

- Test #2: reading of DVM2 should be (3.30.2) V.

- Increase bench load to 23A.

- Test #3: reading of DVM1 should be (1.3020.010) V.

- Decrease bench load to 0A.

- Take off GMUXSEL jumper.

- Set following jumpers: VID3=OFF, VID1=ON, VID0=ON.

- Test #4: reading of DVM1 should be (1.1350.010) V.

- Take off DPSLP# jumper.

- Test #5: reading of DVM1 should be (1.0840.010) V.

- Put DPRSLPVR jumper on.

- Put GMUXSEL jumper on.

- Set following jumpers: VID4=OFF, VID3=ON, VID0=OFF.

- Test #6: reading of DVM1 should be (0.8500.010) V.

- Take off VRON jumper.

- Turn off EXTVCC supply.

- Turn off +3.3V supply.

- Turn off VIN supply.