DC412A INTRODUCTION

DESCRIPTION

Demonstration Circuit DC412A is a synchronous step-down regulator using the LTC3713 No RSENSETM  switching regulator controller.  It can operate at low input voltage down to 1.7V and supply up to 10A of output current.  Higher current can be obtained by changing external components.  DC412A provides a low input buck regulator to supply lower voltages for microprocessors, DSPs, and bus termination applications.  An onboard boost converter generates the gate-drive voltage for the external Power MOSFETs and allows the use of standard 5V logic-level N-Channel MOSFETs.  Gerber files for this circuit board are available.  Call the LTC factory.

JP1 sets the operation mode.  The lower position selects the force continuous synchronous operation, which minimizes noise and RF interference, also allows the output to sink current for applications like bus terminators.  RSNS1, RSNS2, R7, R8 and D1 are there to ease the design modification for symmetric current limit.  Please refer to the “Applications Requiring Symmetric Current Limit” Section in the data sheet for details.

# QUICK START GUIDE

1. Refer to Figure 1 for board orientation and proper measurement equipment setup.

2. Connect the input power supply to the VIN and GND terminals on the top, center of the board.  Don’t increase VIN over 6V (If higher input voltage is desired, use higher voltage rating input capacitors.  If the input voltage is always higher than 4V, you probably want to consider DC378, which uses LTC1778.)

3. Connect the desired loads between VOUT and PGND terminals on the board.  Soldered wires should be used to achieve optimum performance.

4. Shunt JP2 and JP4 for operation.  VOUT goes to 0V with either JP2 or JP4 open.  If JP3 is connected, overcurrent latchoff is disabled.  Refer to the APPLICATIONS INFORMATION in the data sheet for details.

5. Set the FCB Jumper JP1 to the lower position to force continuous synchronous operation at light loads; or to the upper position to enable discontinuous mode operation at light loads.

6. When measuring the output ripple, see Figure 2 for proper scope probe technique.

Figure 1. DC412A Hookup Diagram

# PERFORMANCE SUMMARY

| PARAMETER | CONDITION | VALUE |
| --- | --- | --- |
| Input Voltage Range |  | 1.7V to 6V |
| Maximum Output Current | VIN = 1.7V to 6V | 10A |
| Outputs (Note 1) | 0 to 10A | 1.25V  3% |
| Typical Output Ripple | 10MHz BW; 10A Load; VIN = 3.3V | 15mVP-P |
| Typical Load Regulation | IOUT = 0A to 10A; VIN = 3.3V | -10mV |
| Efficiency | VIN = 3.3V; VOUT = 1.25V; IOUT = 10A | 83% |

Note 1: Higher output current can be achieved by using higher current rating inductor and adding parallel MOSFETs (M3, M4).