## REQUEST FOR FINAL FABRICATION

- BOARD SIZE: 2.50” X 3.70”.   0.008” MIN COPPER THICKNESS, 4 LAYER BOARD.  ROUTED.

LIST OF GBR FILES:

- 475A2-1.PHO & .REP		LAYER 1 - TOP LAYER

- 475A2-2.PHO & .REP		LAYER 2 - GND PLANE

- 475A2-3.PHO & .REP		LAYER 3 - ROUTING SIGNALS

- 475A2-4.PHO & .REP		LAYER 4 - BOTTOM LAYER

- 475A2-TS.PHO & .REP		TOP SILKSCREEN

- 475A2-BS.PHO & .REP		BOTTOM SILKSCREEN

- 475A2-TM.PHO & .REP		TOP SOLDER MASK

- 475A2-BM.PHO & .REP		BOTTOM SOLDER MASK

- 475A2-TP.PHO & .REP		TOP SOLDER PASTE

- 475A2-BP.PHO & .REP		BOTTOM SOLDER PASTE

- 475A2FAB.PHO & .REP		DRILL DRAWING LAYER (FAB DWG)

- 475A2DRL01.DRL			DRILL FILES

- 475A2DRL01.LST			DRILL FILES

- 475A2DRL01.REP			DRILL FILES

- PANELIZE 18 BOARDS ON A 14” X 17” PANEL (6 X 3 ARRAYS),  .020” AIRGAP BETWEEN BOARDS.  TOTAL OF 252 BOARDS ON 14 PANELS.

- MATERIAL: FR4, .062” THICKNESS, 1 OZ CU, TIN-LEAD REFLOW FINISH.

- PLEASE LOOK AT THE FABRICATION DRAWING FOR THE OTHER REQUIREMENTS FOR THE JOB.

- PLEASE CONTACT KIM TRAN AT  408-954-8400 X 3411 IF YOU HAVE ANY QUESTION.

THANK YOU

KIM