LTC3830-1

# Description

Demonstration circuit 493 is a low voltage, 8A DC/DC converter using LTC3830-1 high power, high efficiency step-down switching regulator controller optimized for low voltage applications. Typical applications include CPU power supplies, multiple logic supply generators and distributed power applications. The input voltage of the LTC3830-1 can range from 3V to 8V. The LTC3830-1 uses a synchronous switching architecture with N-channel MOSFETs. The fixed switching frequency is about 200kHz. The LTC3830-1 is a voltage mode feedback controller with a maximum duty cycle of >91% over temperature. A precision internal reference and feedback system provide ±1% output regulation over temperature, load current and line voltage variations. The LTC3830-1 differs from the LTC3830 S8 version by replacing the shutdown function with a soft-start function. See Figure 2 for efficiency curves (1.8V output with optional D2).

Design files for this circuit board are available. Call the LTC factory.

# Quick Start Procedure

Demonstration circuit 493 is easy to set up to evaluate the performance of the LTC3830-1. Refer to Figure 1 for proper measurement equipment setup and follow the procedure below:

- Connect a turned-off input power supply (3V–8V) to the Vin and GND terminals.

- Connect the load between the Vout and GND terminals. The maximum load current is 8A. (RLOAD(MIN) = 0.225 for 1.8V output)

- Once all of the connections are secure, turn on the input power supply. The output voltage should be approximately 1.8V. Be sure to measure the output voltage at the terminals.

| - Proper Measurement Equipment Setup | - Efficiency |
| --- | --- |