LT3421                                                                                                  LTC3421

# Description

Demonstration Circuit 565 is a constant-frequency synchronous boost converter using the LTC3421. Guaranteed to start-up at 1V input voltage, this converter provides up to 95% efficiency.  For 1.8V minimum input voltage, this converter can provide up to 1A load current.

This converter has true output disconnect by eliminating body diode conduction of the internal PMOS rectifier.  This feature reduces the inrush current during start-up and isolates the load from input source in shutdown to eliminate any leakage current.  With the VREF output and low-battery comparator disabled, the IC only draws less than 1A in shutdown.

The output voltage is set at 3.3V. A different output voltage in the range of 2.4V to 5.25V can be obtained by changing one of the feedback resistors. The switching frequency is set at 1MHz, which is a good trade-off between efficiency and size.  The frequency can be adjusted by selecting R13 at RT pin.  It can also be synchronized to an external clock for noise-sensitive applications.

Long wires from input sources (such as wall adaptors) can cause large voltage spikes during initial plug-in.  C12 is installed on DC565 to damp the possible voltage spikes.  C12 is not required for applications where input source is close to the regulator.  Please refer to Application Note 88 for details.

For VOUT > 4.3V applications, D1, Q1, R18 and C10 have to be installed to limit the peak switch voltage below the absolute maximum rating.  Instead of using a schottky diode connected directly from SW pin to the output, this active snubber keeps the LTC3421 true output disconnect feature.  R18 can be used to limit the current going through Q1 in short-circuit conditions.

The optional circuit at ILIM pin can be added to provide current limit foldback for extended short-circuit conditions.  Refer to Short Circuit section in the datasheet for details.

Design files for this circuit board are available. Call the LTC factory.

- Performance Summary (TA = 25°C unless otherwise noted)

| PARAMETER | CONDITION | VALUE |
| --- | --- | --- |
| Minimum Input Voltage |  | 1V |
| Output Voltage VOut | VIN = 1.8V to 3.2V, IOUT = 0A to 1A | 3.3V ±3% |
| Maximum Output Current | VIN = 1.8V to 3.2V | 1A |
| Typical Output Ripple VOut | VIN = 2.4V, IOUT = 1A (20MHz BW) | 30mVP–P |
| Typical Switching Frequency |  | 1MHz |
| Efficiency | VIN = 2.4V, IOUT = 0.2A VIN = 2.4V, IOUT = 1A | 95% Typical 88% Typical |
| On/Off Control | Logic Low Voltage-Off | 0.25V MAX |
|  | Logic High Voltage-On VOUT = 0V VOUT > 2.4V | 1V MIN 0.6V MIN |

# Quick Start Procedure

Demonstration circuit 565 565 is easy to set up to evaluate the performance of the LTC3421LTC3421. Refer to Figure 1 for proper measurement equipment setup and follow the procedure below:

- When measuring the input or output voltage ripple, care must be taken to avoid a long ground lead on the oscilloscope probe. Measure the input or output voltage ripple by touching the probe tip directly across the Vin or Vout and GND terminals. See Figure 2 for proper scope probe technique.

- Place jumpers in the following positions:

JP1	ON or OFF

JP1

RUN

JP2

JP3	AUTO ON

- With power off, connect the input power supply to Vin and GND.

- Turn on the power at the input.

  - Make sure that the input voltage does not exceed 3.3V.

- Check for the proper output voltage. Vout = 3.2V to 3.4V.

  - If there is no output, temporarily disconnect the load to make sure that the load is not set too high.

- Once the proper output voltage is established, adjust the load within the operating range and observe the output voltage regulation, ripple voltage, efficiency and other parameters.

# Graphics

- Proper Measurement Equipment Setup

- Measuring Input or Output Ripple

Figure 3. Load Transient Response (Vin=3V; Channel 1: Vout; Channel 2: Iout: load step from 100mA to 1A)