REQUEST FOR FINAL FABRICATION

- BOARD SIZE: 3.45”” X 3.25””.   0.008” MIN COPPER THICKNESS, 4 LAYER BOARD.

LIST OF GBR FILES		NAME				 PANELIZE

- 671A2-1.PHO & .REP		LAYER 1-TOP LAYER		Y

- 671A2-2.PHO & .REP		LAYER 2-GND PLANE 1		Y

- 671A2-3.PHO & .REP		LAYER 3-GND PLANE 2		Y

- 671A2-4.PHO & .REP		LAYER 4-BOTTOM LAYER	Y

- 671A2-TS.PHO & .REP		TOP SILKSCREEN			Y

- 671A2-BS.PHO & .REP		BOTTOM SILKSCREEN		Y

- 671A2-TM.PHO & .REP		TOP SOLDER MASK		Y

- 671A2-BM.PHO & .REP		BOTTOM SOLDER MASK		Y

- 671A2-TP.PHO & .REP		TOP SOLDER PASTE		Y

- 671A2-BP.PHO & .REP		BOTTOM SOLDER PASTE		Y

- 671A2DRL01.DRL		DRILL FILE				Y

- 671A2DRL01.LST		DRILL FILE

- 671A2DRL01.REP		DRILL FILE

- 671A2FAB.PHO & .REP		DRILL DRAWING LAYER		N

- PANELIZE 6 BOARDS ON 8” X 11” PANEL.  TOTAL OF 408 BOARDS ON 68 PANELS.

- ADD 3 TOOLING HOLES (.125” DIA., UNPLATED) AND 3 FIDUCIALS (.030” DIA.) ON THE COMPONENT SIDE AT 3 CORNERS OF THE PANEL.  CLEAR SOLDER MASK AROUND THE FID. AT LEAST 0.150”DIA.   KEEP AT LEAST .250” DISTANCE FROM THE TOOLING HOLES OR THE FID. TO THE EDGE OF THE BOARD.

- PUT THE NAME OF EACH LAYER ON THE FILM BUT OUTSIDE THE PANEL IF APPLICABLE.

- PLEASE LOOK AT THE FABRICATION DRAWING FOR THE OTHER REQUIREMENTS FOR THE JOB.

- PLEASE CONTACT KIM TRAN AT LINEAR TECH. 408-954-8400 X 3411 IF YOU HAVE ANY QUESTION.

THANK YOU

KIM