Document No.	:	18-073736-01 Rev A

Title			:	ADL8100-EVAL1Z Customer Evaluation Board Test Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| B | ECR-113965 | Initial Release | 04/16/2023 | Love Syl Venenoso |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
| Product Engineer | Love Syl L. Venenoso |

| Model Name | C-Spec | Tester Program |
| --- | --- | --- |
| ADL8100-EVAL1Z | C-11149 A | ADL8100_EVAL1Z.par |

VDD (PS1)

Network Analyzer

Note: Not drawn to scale.

Test 1: S-Parameter Sweep

Equipment Needed:

1 Power Supply – Keithley 2400 or its equivalent

1 2-port Network Analyzer – N5245B or its equivalent

Setup:

Connect the Network Analyzer as shown

Port 1 – RFIN

Port 2 – RFOUT

Network Analyzer Settings:

Min/Max Frequency: 10 MHz to 22 GHz

Number of Points: 501 points

Input Power: -25 dBm

Setting up notes:

- Calibrate the Network Analyzer to compensate for the loss of both input and output cables.

- Record the S11, S12, S21(Gain), S22 of the DUT. Compare the data with the plot below.

Biasing steps:

- Turn on VDD = +5 V

- Apply the RF signal.

- Log the data.

Power off:

- Turn off the RF signal.

- Turn off VDD.

S21, S11 and S22 plot

Test 2: ATE Setup

Equipment and Accessories Needed:

1 Power Supply – Keithley 2400 or its equivalent

1 Signal Generator – Keysight E8267D or its equivalent

1 External Amplifier – N4985A or its equivalent

1 Power meter – Anritsu ML2496A or its equivalent

1 Power Sensor – Anritsu MA2445D or its equivalent

1 20 dB pad

Connect the setup as shown below.

RFIN

RFOUT

Computer

Power Meter

Amplifier

20 dB pad

Power Sensor

RF Source

VDD (PS1)

Note: Not drawn to scale.

Test Parameters: