# Flash Xilinx KC705 for Power-On Configuration

# Part 1: Creating the MCS file

- Start up iMPACT

- Choose Prepare a PROM File or in the iMPACT Flows panel, double-click PROM File Formatter. The following appears:

- Under Step 1. Choose BPI Flash -> Configure Single FPGA and click the first green arrow

- Set Target FPGA to Kintex 7

- Set Storage Device (Bytes) to 128M (M not K)

- Click Add Storage Device, then click the second green arrow

- Change the Output File Name to something meaningful (like the bit file name but with .mcs)

- Set the Output File Location to where you want the file to go

- Set the Data Width to x16

- Click OK.

- Click OK at the dialog box that says “Start adding device file to Revision 0”

- Navigate to the bit file and select it.

- Click NO at the dialog box that says “Would you like to add another device file to Revison 0?”

- Click OK at the next dialog box

- A large dialog that says MultiBoot BPI Flash Revision Assignment will appear, this is OK even though we aren't doing a MultiBoot BPI. Leave all the stuff inside alone, just click OK.

- Under the Operations menu choose Generate File

- Look for Generate Succeeded. You are done.

# Part 2: Programming the Flash

- Set SW13 so that all pins are down (low, opposite of arrow on side) except pin4

- Plug micro USB cable into connector labled “JTAG”, connect to PC

- Power up board, wait for drivers to install

- Start iMPACT if necessary

- Do a boundary scan to initialize the device chain

- Right click the device icon:

- Click “Add SPI/BPI Flash”

- Browse to the MCS file (see part 1 to generate)

- In the top-right list box choose 28F00AP30

- Change the “Select RS[1:0]b Pin Address Bits” list box to 26:25

- Click OK.

- Right-lick the new “Flash” box in the device Icon and choose program

- Click OK.

- Wait for programming to finish. You are done.