Document No.	:	18-072506-01 Rev A

Title			:	EVAL-LT3074-AZ Customer Evaluation Board Test Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| A | ECR-122595 | Initial Release |  | Carla Jeremy Jardin |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
| Apps Engineer | Carla Jeremy Jardin |
| Apps Engineer | Andrew Radosevich |

Bench Set Up 1

Record measurements in the Test Data Table and verify that the recorded measurements are within the limits.

Make sure these jumpers are installed and in these positions before and after testing:

- JP1	ON

- JP2	ON

- JP3	VASEL1

- JP4	EXT

Also connect J12 to DC1613A (USB-to-PMBus Controller) dongle and connect to a PC to communicate with the board through PMBus.

NOTES:

- A VIN or VBIAS voltage that is too close to the programmed output voltage (too low) may cause dropout operation and a loss of output voltage regulation. An input voltage that is too high above the output may increase power dissipation to an unacceptable level.

- Ramp the input supplies up and down by hand to avoid overshoot since the LT3074 IC has relatively low abs. max. ratings for VIN and VBIAS.

- With both the VIN and EXTVBIAS supplies turned to 0V and turned off, connect the circuit as shown in the Bench Set Up 1 drawing. Do not connect the load yet.  Make sure the POWER jumper JP1 is in the ON position, the CLOCK jumper JP2 is in the ON position, the ASEL jumper JP3 is in the VASEL1 position and the BIAS MODE jumper JP4 is in the EXT position.

- Turn both input supplies on. Turn up the EXTVBIAS supply to 5V by hand.  Turn up the VIN supply by hand to around 1.5V. Connect the 0.4 Ohm load. Readjust VIN to 1.5V. Measure IOUT, IBIAS and VOUT. IOUT is just VRLOAD/0.4 OHM.  Record the IMON voltage.

- Open the EEPROM.exe program file and choose “EVAL-LT3074 [LDO with PMBUS]”.

- Click the program button a note that shows “PASS!!” should show.

NOTE:

- Steps 3 and 4 must be performed 100% on all assembled boards so the EEPROM can identify the EVAL board properly.

- Open the LTpowerPlay software. Verify that the software automatically detects the EVAL-LT3074-AZ and shows an interface screen that is similar to the one shown below. It may be useful to click on the RESET button at the top of the LTpowerPlay screen.

- Move the POWER jumper JP1 to the OFF position. Measure the output voltage.

- Move the POWER jumper JP1 to the ON position while observing the VBIAS and PG voltages with respect to VO- with an oscilloscope. With this, also check the LEDs for POWER ON (D3) and POWER GOOD (D1). These LEDs should all change from off to green. The ALERT LED (D2) should be turned off since there shouldn’t be any error. Use the VOUT NOISE MONITOR BNC connector as the input to the oscilloscope to monitor VOUT. Measure the output voltage rise time, make sure the VOUT voltage is stable, make sure VBIAS stays constant and make sure PG transitions properly as shown in the Figure 1 scope capture. Then measure the VOUT ripple voltage as shown in Figure 2. Momentarily remove the PMBus cable to reduce noise when observing VOUT ripple.

- Turn down the VIN supply to 0V by hand. Turn down and remove the EXTVBIAS supply. Move the jumper JP4 to 3V3 bias mode. Make sure that the board is still connected to the PC through the DC1613A dongle. Turn up the VIN supply to 1.5V. Measure VOUT.

- Check the address of LT3074 in the LTpowerPlay using the JP4 jumper. Re-enumerate and RESET the PMBus system as necessary by clicking on the appropriate buttons at the top of the LTpowerPlay screen.

See table below:

| VASEL1 | 7h6D |
| --- | --- |
| VASEL2 | 7h6E |
| VASEL3 | 7h6F |

- Check the JP2, the CLOCK jumper if it is working well. Move JP2 jumper to the OFF position. If the board can no longer communicate with LTpowerPlay through PMBus, this means that the CLOCK jumper is turned off and is working well.

- Turn down the VIN supply to 0V. Move the JP3 jumper back to VASEL1, return the JP2 jumper back to the ON position and put the JP4 jumper back to EXT so that all jumpers will return to their original positions.

Test Data Table

Figure 1:  Check Output Voltage Rise Time and Stability.  Check that VBIAS Is Constant and PG Transitions Properly

Figure 2:  VOUT Ripple Voltage