Document No.	:	18-080142 Rev A

Title			:	EVAL-LT3078-AZ Customer Evaluation Board Test Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| A | N/A | Initial Release | 10/03/2023 | Andrew Radosevich |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
| Apps Engineer | Andrew Radosevich |

Bench Set Up 1

Bench Set Up 2

Record measurements in the Test Data Table and verify that the recorded measurements are within the limits.

Make sure these jumpers are installed and in these positions before and after testing:

- JP1	ON

- JP2	FLOAT

- JP3	FLOAT

- JP4	HIGH

- JP5	5.5A

- JP6	OPEN

NOTES:

- A VIN or VBIAS voltage that is too close to the programmed output voltage (too low) may cause dropout operation and a loss of output voltage regulation.  An input voltage that is too high above the output may increase power dissipation to an unacceptable level.

- Ramp the input supplies up and down by hand to avoid overshoot since the LT3078 IC has relatively low abs. max. ratings for VIN and VBIAS.

- With both the VIN and VBIAS supplies turned to 0V and turned off, connect the circuit as shown in the Bench Set Up 1 drawing.  Do not connect the load yet.  Make sure the POWER jumper JP1 is in the ON position, the CURRENT LIMIT jumper JP5 is in the 5.5A position and the MARG jumper JP6 is in the OPEN position.  Make sure the VO2 JP2, VO1 JP3and VO0 JP4 jumpers are in the following positions to program 1.2VOUT:

- JP2	FLOAT

- JP3	FLOAT

- JP4	HIGH

- Turn both input supplies on.  Turn up the VBIAS supply to 5V by hand.  Turn up the VIN supply by hand to around 1.5V.  Connect the 0.24 Ohm load.  Readjust VIN to 1.5V.  Measure IOUT, IBIAS and VOUT.  IOUT is just VRLOAD/0.24 OHM.  Record the IMON voltage.  Verify that the TEMP voltage increases as the IC heats up.

- Move the POWER jumper JP1 to the OFF position.  Measure the output voltage.

- Move the POWER jumper JP1 to the ON position while observing the VBIAS and PG voltages with respect to VO- with an oscilloscope.  Use the VOUT NOISE MONITOR BNC connector as the input to the oscilloscope to monitor VOUT.  Measure the output voltage rise time, make sure the VOUT voltage is stable, make sure VBIAS stays constant and make sure PWRGD transitions properly as shown in the Figure 1 scope capture.  Then measure the VOUT ripple voltage as shown in Figure 2.

- Turn down the VIN supply to 0V by hand.  Remove the 0.24 Ohm resistor load.  Move the CURRENT LIMIT jumper JP5 to the 2.5A position.  Move the VO2 JP2, VO1 JP3and VO0 JP4 jumpers to the following positions to program VOUT to 0.5V:

- JP2		LOW

- JP3		LOW

- JP4		LOW

Connect a constant current load that is turned down to 0A.  Turn up the VIN supply by hand to around 0.8V.  Increase the constant current load to 2A.  Readjust VIN to 0.8V.  Measure IBIAS and VOUT.  Record the IMON voltage.

- Move the CURRENT LIMIT jumper JP5 to the INTERNAL position. Record the IMON voltage.

- Move the CURRENT LIMIT jumper JP5 back to the 5.5A position. Move the MARG jumper JP6 to the GND position.  Measure VOUT.

- Move the MARG jumper JP6 to the VBIAS position.  Measure VOUT.

- Return the MARG jumper JP6 to the OPEN position.  Turn down both the VIN and VBIAS supplies to 0V.  Return the VO2 JP2, VO1 JP3and VO0 JP4 jumpers to the following positions to program the output to 1.2V:

- JP2	FLOAT

- JP3	FLOAT

- JP4	HIGH

- Remove everything connected to the board.  Measure the resistance to ground of the R4 pad indicated in the Bench Set Up 2 drawing.

- Connect a 5V power supply between the Q1_G test point and ground and measure the resistance to ground of the R4 pad again.

- Disconnect the 5V power supply.

Test Data Table

Figure 1:  Check Output Voltage Rise Time and Stability.  Check that VBIAS Is Constant and PG Transitions Properly

Figure 2:  VOUT Ripple Voltage