Document No.	:	18-081769-01 Rev A

Title			:	EVAL-LT83402-AZ Customer Evaluation Board Test Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| A | N/A | Initial Release | 10/2025 | ERIK LAMP |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
| Apps Engineer | ERIK LAMP |

INITIAL SETUP NOTES:

- The test requires the following equipment:

  - A DC supply that source at least 12V and 2A.

  - An electronic load that can sink at least 2.5A at 3.3V.

  - 4x DMMs.

- Make sure the JP1 jumper is in the FCM position.

- Power supply and electronic load off.

- Connect 1x DMM as a current meter from the power supply “+”  to “VIN” (E6), as shown in the figure.

- Connect 1x DMM as a voltmeter to “VIN SENSE” (E3) and “GND SENSE” (E4), as shown in the figure.

- Connect 1x DMM as a voltmeter to “VO SENSE” (E10) and “GND SENSE” (E11), as shown in the figure.

- Connect the “-“ of the power supply to the “GND” (E7), as shown in the figure.

DEMO CIRCUIT REGULATION TEST:

- Set the power supply to 12V and turn on the power supply.

- The output voltage should read 3.32V (3.29V~3.35V).

- Connect 1x DMM as a current meter from “GND” (E13) to the “-” of the electronic load, as shown in the figure.

- Connect the “VOUT” (E12) of the board to the “+“ of the electronic load, as shown in the figure.

- If the electronic load is not connected correctly, it will either protect itself and prevent from being turned on, or it could get damaged.

- Turn on the electronic load with the current set to 0A.

- Then increase the load to 2.5A, with Vin still set at 12V; the output voltage should be about 3.32V.

- Turn off the electronic load, then turn off the power supply.

| Test Conditions |  | Accept/Reject |  |
| --- | --- | --- | --- |
| VIN | IOUT | VOUT |  |
| 12V | 0A | 3.29V ~ 3.35V |  |
| 12V | 2.5A | 3.29V ~ 3.35V |  |