MECHANICAL DESCRIPTION OF BOARD

MAX14548E EVALUATION KIT+ REV-A

| Material: | RoHS Compliant FR408 Laminate material compatible with Lead-Free soldering processes |
| --- | --- |
| Size: | 2.400” X 4.000” |
| Thickness: | 0.062” |
| Layers: | 4 |
| Solder Mask: | GREEN LPI, TAIYO PSR-4000 |
| Solder Mask Aperture Tolerance: | +/-0.0005” |
| Legends: | White. Single-sided. (clipped all legends from exposed metal) |
| Partial Vias: | Partial vias should be filled with via fill and capped. Surface mount pad plating must be flat. Laser drilled partial vias from Layer 1-2 and Layer 2-3 Via Name : 1-2 Via Count : 24 Via Name : 2-3 Via Count : 6 |
| Copper Clad: | 1OZ minimum for all layers |

VENDOR LOGO & DATE CODE:	Allowed only in Ink on bottom side only.

| PLATING | Must be Lead free and RoHS compliant |
| --- | --- |
| Finish: | Immersion Gold |
| Thru Holes: | 0.001” min |
| Quality: | Manufactured in accordance with IPC-A-600 |

SURFACE MOUNT PADS:		225

TOLERENCES:

| Board Dimensions | +/- 0.010” |
| --- | --- |
| Plated-Thru Holes: | +/- 0.003” |
| Pattern-to Pattern | +/- 0.003” |
| Legend to Legend | +/- 0.007” |

Drill Schedule is located on the Drill Plot.

Johann Gualberto

Maxim Integrated Products

120 San Gabriel Drive, MS165

Sunnyvale, CA 94086

Phone	(408) 470-6910

Fax	(408) 530-2482

Email:	JOHANN.GUALBERTO@MAXIM-IC.COM

10/20/2009