Functional Design Specification

SmartMesh-IA Radio Duty Cycle

Document Number:

- 055-0028

- Rev:

- Version 1.0, Draft 1

Date:

February 15, 2008

Confidentiality

This document is provided under a Non-Disclosure Agreement solely for the purpose of enabling the international radio certification of Dust Networks™ products.  Distribution of this document for any other purpose is prohibited.  Distribution of this document shall be completed in a manner such that the material content of this document shall remain confidential by all parties.

Trademarks

Dust Networks™, the Dust Networks logo, SmartMesh-XR™, SmartMesh-XT™, SmartMesh-XD™, mesh-to-the-edge™, and Mote-on-Chip™ are trademarks of Dust Networks, Inc. Dust® and SmartMesh® are registered trademarks of Dust Networks, Inc. All third-party brand and product names are the trademarks of their respective owners and are used solely for informational purposes.

Copyright

This documentation is protected by United States and international copyright and other intellectual and industrial property laws. It is solely owned by Dust Networks, Inc. and its licensors and is distributed under a restrictive license. This product, or any portion thereof, may not be used, copied, modified, reverse assembled, reverse compiled, reverse engineered, distributed, or redistributed in any form by any means without the prior written authorization of Dust Networks, Inc.

RESTRICTED RIGHTS: Use, duplication, or disclosure by the U.S. Government is subject to restrictions of FAR 52.227-14(g) (2)(6/87) and FAR 52.227-19(6/87), or DFAR 252.227-7015 (b)(6/95) and DFAR 227.7202-3(a), and any and all similar and successor legislation and regulation.

© Dust Networks, Inc. 2006, 2007, 2008. All Rights Reserved.

# Determination of Radio Duty Cycle

Dust radios operate on a TDMA time schedule that uses 10 ms timeslots. A transmit timeslot consists of 5 stages:

- Initialization: radio is prepared for transmit (transmitter is off)

- Ramp: transmitter is ramped to peak power

- Transmit: 128 bytes of data maximum + 5 bytes preamble/SFD

- Turnaround: radio is set to receive

- Receive: radio waits in receive for ACK, then turns off

Total transmit time for a 128 byte packet plus all overhead (SFD / Preamble / Ramp) is 4.34ms, shown in the figure below:

Maximum duty cycle is therefore:

4.34 ms on / 10 ms = 43.4 %