##### GENERAL SPECIFICATION <Product Name>

|  |  | #### Applicant Statement |
| --- | --- | --- |
| 1. | Necessary Bandwidth : | 2.7 MHz |
| 2. | Classification of Emissions | 2G40P7D |
| 3. | RF Channel Spacing | 5 MHz |
| 4. | Rated Duty Cycle : Choose from : Handheld, Mobile , Base Repeater, etc. | Max: <43.4% Typical: <1% |
| 5. | Battery Powered : Yes/No |  |
| 6 | Nominal Voltage |  |
| 7 | Operating Humidity |  |

TRANSMITTER SPECIFICATION

|  |  | #### Applicant Statement |
| --- | --- | --- |
| 1. | Frequency Band | 2.4 GHz |
| 2. | Test Frequency | 2.4-2.4835 GHz |
| 3. | Modulation Input Port | Integrated |
| 4 | Maximum RF Power Output (As submitted form Approval) | 9mW (+8dBm) |
| 5 | Maximum RF Power Output (Manufacturer Specified) | 9mW (+8dBm) typical |
| 6 | Out of Band Emissions | N/A |
| 7 | Spurious Emissions | 2nd harmonic radiations: -54.2 dBm 3rd Harmonic radiations: -50.2 dBm |
| 8 | Modulation Distortion | IEEE 802.15.4 direct sequence spread spectrum (DSSS) |
| 9 | Audio Frequency Response Characteristic | N/A (Data only) |
| 10 | Hum and Noise Level | N/A (Data only) |
| 11 | Maximum Frequency Deviation | 1.25 MHz |
| 12 | Maximum RF Frequency Error | Frequency Stability: +/- 96 kHz |

RECEIVER SPECIFICATION

|  |  | #### Applicant Statement |
| --- | --- | --- |
| 1. | Frequency Band | 2.4 – 2.4835 GHz |
| 2. | Receiver Test Frequency | Programmable: 2405+n*5 MHz, (n=0..14) |
| 3. | Rated Audio Output Power | N/A (Data only) |
| 4 | Maximum Audio Output Power | N/A (Data only) |
| 5 | ##### Speaker Impedance | N/A (Data only) |
| 6 | Value of resistive load into which the audio output is delivered | N/A (Data only) |
| 7 | I.F. Frequencies | Intermediate Frequency: 2.5 MHz |
| 8 | Useable sensitivity | -92.5dBm typical |
| 9 | Audio Frequency Response Characteristics | N/A (Data only) |
| 10 | Adjacent Channel Selectivity | 15 dBc |
| 11 | Offset Channel Selectivity | 30 dBc |
| 12 | Spurious Response Attenuation | N/A |
| 13 | Intermodulation Spurious Response Attenuation | N/A |
| 14 | Antenna Conducted Spurious | N/A |

#### DECLARATION BY APPLICANT

I/WE declare that all information given herein are to the best of my.our knowledge, true and correct

Authorised Officer’s Signature

Full Name of Signatory		:

Designation				:

Date					: