Intelligent Lighting Controller User Guide

Author:	Marko Kannisto and Philip Simpson

User Guide revision and change history:

| Revision | Date | Changes made |
| --- | --- | --- |
| 01 | 22.01.2012 | First release (Light_Controller_SW_rev_01.hex) |

CONTENT

1.	General information about the system………….…………………………..	2

2.	User Interface……………………………………………………………….	2

3.	Setting parameters…………………………..………………………………	3

4.	Reading parameters……....………………………………………………...	3

5.	Using the Manual-Auto switch……………………………………………...4

6.	Reading the software revision / resetting the system……………………….	4

1.	General information about the system

Intelligent lighting controller measures ambient light level and tracks time. Lux threshold, switch-off time at the evening and switch-on time in the morning are programmable.

Lux threshold can be programmed from 0 lux to 20 lux in single lux steps.

Switch-off time at the evening can be programmed from 20.00 – 04.45 in 15 minute steps. If switch-off time is set as CO (= continuous), system is functioning as a dark switch (light is switched on when ambient light level is equal or lower than lux threshold, and switched off when ambient light level is greater than lux threshold).

Switch-on time in the morning can be programmed from 05.00 – 11.45 in 15 minute steps.

All parameters are stored in the Microcontroller’s non-volatile EEPROM memory.

2.	User Interface

User interface consists of one push-button, two 7-segment LED displays and Auto/Manual switch. There are also four indicator LEDs: V_ON, OK, HR and MIN.

V_ON-LED (D2) indicates if system is powered. OK-LED indicates system state: fast blinking indicate that system is in Auto-mode, slower blinking indicate that system is in Manual-mode (relay continuously on).

HR-LED and MIN-LED indicates if hour (HR) or minute (MIN) are displayed. When relay is on, HR- and MIN-LEDs are switched on simultaneously.

After ambient light measurement HR- and MIN-LED blinks three times, and then the measured lux value is displayed. If lux value is greater than 100, “FF” is displayed.

3.	Setting parameters

Note: To set time and other parameters, Manual/Auto-switch must be in Auto.

System uses 24-hour clock. Current time, lux threshold, switch-off time at the evening and switch-on time in the morning can be recorded to system memory following these steps:

- Press SET-button until HR- and MIN LEDs lit.

- Release SET-button.

- Set current hour with SET-button. When hour is set, keep SET-button pressed until OK-LED blinks. Release SET-button.

- Set current minute with SET-button. When minute is set, keep SET-button pressed until OK-LED blinks. Release SET-button.

- Set lux threshold with SET-button. When threshold is set, keep SET-button pressed until OK-LED blinks. Release SET-button.

- Set switch-off hour at the evening with SET-button. When switch-off hour is set, keep SET-button pressed until OK-LED blinks. If system is meant to be used as a dark switch, set off-hour to CO = COntinuos mode. If CO is set, parameter setting ends here.

- Set switch-off minute at the evening with SET-button. When switch-off minute is set, keep SET-button pressed until OK-LED blinks. Release SET-button.

- Set switch-on hour in the morning with SET-button. When switch-on hour is set, keep SET-button pressed until OK-LED blinks. Release SET-button.

- Set switch-on minute in the morning with SET-button. When switch-on minute is set, keep SET-button pressed until OK-LED blinks.

After parameter settings, system will display all parameters in the same order as they were recorded.

4.	Reading parameters

Note: To read parameters, Manual/Auto-switch must be in Auto.

Parameters can be read by pressing SET-button shortly (less than 1 second). Parameters are then displayed in following order:

- Current hour,

- current minute,

- lux threshold,

- switch-off hour,

- switch-off minute,

- switch-on hour,

- switch-on minute.

5.	Using the Manual-Auto switch

Relay can be turned on manually by setting the Manual/Auto switch to Manual. When in Manual mode, OK-LED is blinking slowly and HR- and MIN-LED are on simultaneously.

When user wants to use the system in Auto mode, set Manual/Auto switch to Auto. Please note that wall light switch must be turned on, otherwise system can not switch luminaire(s) on.

6.	Reading the software revision and resetting the system

Reading the software revision is done by keeping the SET-button pressed for about 5 seconds (in Auto mode). Software revision is then shown on display for 5 seconds time.

If user wants to reset the system, press SET-button during the above mentioned 5 second time period (when software revision is shown on the displays). Then system is being resetted. Reset doesn’t erase any information from Microcontroller memory. Because there is only one button in the user interface, this is a good and safe way to perform system reset if that is needed.