a

HSC - Greensboro      7910 Triad Center Drive, Greensboro, North Carolina    27409

20 Sept 2005

AD9851FS EVALUATION BOARD

Buyer:     	RICHARD FLEMING

Draftsmen:  	TED HECHT

Phone:	336-605-4039 		FAX:	336-605-4187

Data Format

o Data format in Gerber using absolute coordinates.

o Leading zeros are omitted.

o End of block character is *.

o Digits are in integer format representing thousandths of an inches.

o Board dimensions are 3.000 by 4.000 inches.

o Board offset is x: 0 y: 0 inches.

o Drawing is scaled to 1:1.

o Plot filename:         Description:

art01 .PHO        	TOP CIRCUIT LAYER 1

art02.PHO        	GND PLANE LAYER 2

art03.PHO        	PWR PLANE LAYER 3

art04.PHO        	BOTTOM CIRCUIT LAYER 4

sst0126.PHO         	SILK-SCREEN TOP LAYER

ssb0429.PHO         	SILK-SCREEN BOTTOM LAYER

Sm0121.PHO        	SOLDERMASK TOP LAYER

Sm0428.PHO         	SOLDER MASK BOTTOM LAYER

Dd0124.PHO         	DRILL DETAIL

page 1 of  3

# To                DON AYERS                                       From           STEVEN LITTRELL

Co.              Printed Circuit Solutions                                        ANALOG DEVICES

Phone #       (540) 236-4921                                     Phone #        336-605-4109

Fax #	        (540) 236-2458                      	          Fax #            336-605-4187

a

HSC - Greensboro      7910 Triad Center Drive, Greensboro, North Carolina    27409

28 Oct 1998

AD9851 EVALUATION BOARD

GS 00516 Rev A

P.O. # GA

Buyer:     	RICHARD FLEMING

Draftsmen:  	TED HECHT

Phone:	336-605-4039 		FAX:	336-605-4187

Material Specifications

o Board Material: G10-FR4

o Finished board thickness: .062 inches.

o Finished copper weight: 1  oz.

NOTE:  Base material can be plated to this thickness.

Finish Specifications

o Board finish: Solder mask on solder plate.

o Solder mask on both sides from artwork supplied per finish above using

Flat Blue solder mask.

o Silk screen on both sides using non-conductive white epoxy ink.

o Total PWB area is  12.0 sq. inches.

o 4 layer board.

o Modify included Excellon drill tape to include house references standards.

o 148 holes using 6 different drill sizes.

o Drill size stated applies after plating.

o please quote both  5 day and 10 day service.

o please quote 100 boards for each turn time.

o Quality issues should reference IPC-A-600.

SEE BOTTOM LEFT CORNER OF FILM FOR LAYER ORIENTATION.

SEE  ACCOMPANYING DRILL DETAIL DRAWING FOR BOARD DIMENSIONS.