AD8349 Customer Evaluation Board Test Procedure

Equipment required:

- Voltage supply/supplies capable of delivering:

  - +5 V 200 mA

  - -5 V 100 mA

  - 400 mV

- One RF signal generator capable of generating 1900 MHz @ -6 dBm.

- One low frequency signal generator capable of generating a
1 MHz sinewave at 2 Vpp with a 0 V dc bias.

- One ADI A00305A single-ended to differential board.

- Assorted jumper wires and SMA cables.

Connections

# Supplies

- Single-ended to differential board

  - Connect supply grounds to AGND test point or GND jack

  - Connect +5 V supply to VP1

  - Connect –5 V supply to VN1

  - Connect 400 mV supply to I_IN_DCCM

  - Ensure that I_IN-DCCM and Q_IN_DCCM are shorted to each other on underside of board

- AD8349

  - Connect +5 V supply to VPOS test point (TP3)

  - Connect supply ground to GND test point (TP1).

# Baseband

    - From 1 MHz source, split two ways, to I_IN_AC and Q_IN_AC

    - Between single-ended to differential board and AD8349 eval board

  - IP to IP

  - IN to IN

  - QP to QP

  - QN to QN

# RF

    - From RF signal generator to LO SMA connector on AD8349 eval board

    - From VOUT SMA connector on AD8349 eval board to spectrum analyzer input

## Setup

- Ensure W2 and W3 are jumpered

- Set RF signal generator to 1900 MHz at -6 dBm

- Set baseband sinewave generator to 1 MHz with 2 Vpp and 0 V dc bias

- Verify with an oscilloscope (using a probe) that peak to peak voltage of the sinewave at IP, IN, QP and QN is 600 mV and that the dc bias is 400 mV.

  - If not, adjust baseband signal generator voltage to obtain the proper amplitude of 600 mVpp.

  - If bias is off, adjust 400 mV dc supply voltage.  Offset between IP and IN (and QP and QN) should be negligible.

Test Procedure

    - Turn on power supply sources

    - With SW1 in the B (off) position, measure supply current

  - Current should be between 130 mA and 145 mA.  Pad these numbers up by 45 mA if A00305A board current is included.

    - Move SW1 to the A (on) position

  - Current should be between 135 mA and 150 mA.  Pad these numbers up by 45 mA if A00305A board current is included.

- Turn on baseband and RF generators

- Verify that a spectrum similar to that show below is obtained.

- Verify that the peaks of the signals fall within the following limits:

|  | # LSB | LO | USB |
| --- | --- | --- | --- |
| Frequency | 1899 MHz | 1900 MHz | 1901 MHz |
| Upper Limit | None | -35 dBm | None |
| Lower Limit | -2 dBm | None | -2 dBm |

- If all conditions above are satisfied then board passes

- If not, attempt to troubleshoot until board passes.  If still unable to determine cause of failure, label board with failure mechanism and return for further testing.

- Return SW1 to B position and END TEST.