a

HSC - Greensboro      7910 Triad Center Drive, Greensboro, North Carolina    27409

3/30/2005

AD9959 Evaluation Board, Rev. C

Buyer: Randall Hudson /Bill Lanier

Designer/Draftsman: Randall Hudson

Phone:	336-605-4224

FAX:	336-662-4432

E-mail:  Randall.Hudson@analog.com

Data Format

- Data format in Gerber using absolute coordinates (274D).

- Leading zeros are omitted.

- End of block character is *.

- Digits are in integer format representing thousandths of an inch.

- Board dimensions are 6.500 by 4.500 inches.

- Board offset is x: 0 y: 0 inches.

- Drawing is scaled to 1:1.

- Plot filename:         		Description:

art01.pho	TOP CIRCUIT LAYER 1

art02.pho	GND PLANE LAYER 2

art03.pho	POWER PLANE LAYER 3

art04.pho	BOTTOM CIRCUIT LAYER 4

sst0126.pho	SILK-SCREEN TOP LAYER

ssb0429.pho	SILK-SCREEN BOTTOM LAYER

sm0121.pho	SOLDERMASK TOP LAYER

sm0428.pho	SOLDER MASK BOTTOM LAYER

dd0124.pho	DRILL DETAIL

drl01.drl	NC Drill thru hole

drl02.drl	NC Drill Layer 1 to Layer 2

drl03.drl	NC Drill Layer 3 to Layer 4

To       		Alice Montgomery		From    	Randall Hudson

Co.      	            PCSM                	 	Co.     	ANALOG DEVICES

Phone #  	(276) 236-4921	           	Phone # 	910-605-4224

Fax #		(276) 236-2458               	Fax #   	910-662-4432

a

HSC - Greensboro       7910 Triad Center Drive, Greensboro, North Carolina    27409

AD9959 Evaluation Board, Rev. C

Buyer:  Randall Hudson /Bill Lanier

# Designer/Draftsman: 	Randall Hudson

Phone:	336-605-4224

FAX:	336-662-4432

# Material Specifications

- Board Material: G10-FR4

- Finished board thickness: .062 inches.

- Finished copper weight: 1/2 oz.

NOTE:  Base material can be plated to this thickness.

- Impedance Matching: Layer 1 to Layer2 & Layer3 to Layer4 with height between the matching layers at .012”

Finish Specifications

- Board finish: Solder mask on bare copper

- Solder mask on both sides from artwork supplied per finish above using

BLUE LPI solder mask.

- Immersion Silver finish.

- Silkscreen on both sides using non-conductive WHITE epoxy ink.

- Total PWB area is 29.25 sq. inches.

- 4 layer board.

- Modify included Excellon drill tape to include house references standards.

- 848 holes using 10 different drill sizes.

- The 488 holes showing a drill size of 12 should be 13.5 hole plated.

- 244 of the  holes showing a drill size of 12 are blind (partial) vias.

- The 145 holes showing a drill size of 14 are blind (partial) vias.

- Drill size stated applies after plating.

- Quality issues should reference IPC-A-600.

- Please quote:	30 boards with a 10-day turn

SEE BOTTOM LEFT CORNER OF FILM FOR LAYER ORIENTATION.

SEE ACCOMPANYING DRILL DETAIL DRAWING FOR BOARD DIMENSIONS.

Please send 1-Up plots back to me, via my e-mail address.

E-mail:  Randall.Hudson@analog.com